# Concept Report Template

## A.1	Federal Oversight – Concept Reports

Full Oversight Projects – The designation for Full Oversight is provided in the Department’s Project Management System under the “Indicators” section. Projects designated as Full Oversight will require FHWA review and approval of the Concept Report. Note that the Federal Highway Administration determines which projects utilizing federal funding will be designated as Full Oversight and the designation is independent of project type. Full Oversight designation was superseded by Projects of Division Interest (PoDI) in 2014.

Projects of Division Interest (PoDI) – The designation for PoDI is provided in the Department’s Project Management System under the “Indicators” section.  If the project is indicated to be a PoDI, the Project Manager will need to review the project Stewardship and Oversight Plan to determine if FHWA is exercising oversight over the Concept Phase prior to submitting the Concept Report for approval. If FHWA oversight of the Concept is noted in the project Stewardship and Oversight Plan, FHWA approval of the Concept Report will be required. Note that the Federal Highway Administration determines which projects utilizing federal funding will be designated as PoDI and the designation is independent of project type.

Exempt - The reference to “Exempt” projects under this definition does not refer to Air Quality exempt projects; these designations relate to FHWA oversight only.

State Funded (SF) - The SF designation is to be selected for projects for which state funds are programmed.

## A.2	Federal Agencies to Invite to Concept Meetings

The Project Manager will extend an invitation to the following Federal Agencies, as appropriate:

| Division Administrator Federal Highway Administration 61 Forsyth Street, SW Suite 17T100 Atlanta, GA 30303 | Regional Administrator Federal Transit Administration 61 Forsyth Street, SW Suite 17T50 Atlanta, GA 30303 |
| --- | --- |

The Office of Environmental Services will extend an invitation, as appropriate, to the following Federal Agencies to attend Concept Meetings:

| Chief of Wetlands Regulatory Section Environmental Protection Agency 345 Courtland Street, NE Atlanta, GA 30365 | Chief of Regulatory Functions Branch U.S. Army Corps of Engineers P.O. Box 889 Savannah, GA 31402 |
| --- | --- |
| U.S. Department of the Interior Fish and Wildlife Service Room 334, Federal Building 801 Gloucester Street Brunswick, GA 31520 | National Marine Fisheries Service Habitat Conservation Division P.O. Box 12607 Charleston, SC 29422 |
| Environmental Protection Agency Region IV 345 Courtland Street, NE Atlanta, GA 30365 |  |

## A.3	General Instruction and Information

- Please use the most current version of the Concept Report when submitting your report. An up-to-date MS-Word version of the blank report is available for download on the R.O.A.D.S. website and/or may be provided by the Office of Design Policy and Support’s Conceptual Design Group for your use upon request.  There are a number of pull-down menus and check boxes available in MS-Word version of the Concept and Revised Concept reports.

- Instructions and information to assist in completing the report are shown in italics for easy identification, and should be removed prior to report submission.

- Remember that the example report is a template and is intended to be flexible.  If changes to the report are needed for a specific project, the engineer of record and Project Manager should exercise their judgment when making changes from the approved format.

- Documentation of QC/QA tasks being performed on the report should be provided when the Concept Report is submitted.

- Reports should be submitted in .pdf format via email to:  ConceptReports@dot.state.ga.us

- Design Variances and Design Exceptions - Please note that FHWA typically requires that Design Variances and Design Exceptions be approved prior to approval of the Concept Report for PoDI projects; for Exempt projects, Design Variances and Design Exceptions are normally requested during the Concept or Preliminary Design Phases.

- Make sure all attachments, maps, layouts, etc. are legible.

- Keep in mind that reports are printed for approval and filing.  Standard page sizes should be utilized – e.g. 8 ½ x 11” (letter); 8 ½ x 14” (legal); and 11 x 17” (ledger).  Please avoid plan size and half size pages.

- Please provide any feedback or questions regarding the Concept Report format to the State Conceptual Design Engineer.

## A.4	Concept Report Template

See following pages.

# DEPARTMENT OF TRANSPORTATION

# STATE OF GEORGIA

# PROJECT CONCEPT REPORT

| Project Type: |  |  | P.I. Number: |  |
| --- | --- | --- | --- | --- |
| GDOT District: |  |  | County: |  |
| Federal Route Number: |  |  | State Route Number: |  |
|  | Project Number: | (if available) |  |  |

Project Description (provide a very brief description of the project; Description should be no more than 2-3 lines long)

Submitted for approval:  (email to “Concept Reports”; remove notes in italics & delete any inapplicable signature lines)

| Consultant Designer & Firm or GDOT Concept/Design Phase Office Head & Office |  | Date |
| --- | --- | --- |
| (if applicable) |  |  |
| Local Government Sponsor |  | Date |
| State Program Delivery Engineer |  | Date |
| GDOT Project Manager |  | Date |

# Recommendation for approval: (Delete any inapplicable signature lines)

| State Environmental Administrator |  |  | Date |
| --- | --- | --- | --- |
| State Traffic Engineer |  |  | Date |
| Project Review Engineer |  |  | Date |
| State Utilities Engineer |  |  | Date |
| District Engineer |  |  | Date |
| (if applicable) |  |  |  |
| State Bridge Engineer |  |  | Date |
| ☐ | MPO Area: This project is consistent with the MPO adopted Regional Transportation Plan (RTP)/Long Range Transportation Plan (LRTP). |  |  |
| ☐ | Rural Area: This project is consistent with the goals outlined in the Statewide Transportation Plan (SWTP) and/or is included in the State Transportation Improvement Program (STIP). |  |  |
| State Transportation Planning Administrator |  |  | Date |

PROJECT LOCATION MAP

Include a project location map sufficient to clearly locate the project and its beginning and ending point.

PLANNING AND BACKGROUND

Project Justification Statement:  A brief statement provided by the Office of Planning, Office of Bridge Design, or the Office of Traffic Operations, identifying and explaining the major issue(s) that the project is intended to address.  The Project Justification should include:

- Name of the office that prepared or approved the Project Justification Statement.

- Any designated programs that the project is included in (e.g. GRIP, SRTS, STRAHNET, Oversized Truck Route, designated bike route, APD, etc.). How the project originated - for example: Transportation Board, Senior Management,  PNRC, Planning Office, planning study, local government, MPO,  Operations, Bridge Maintenance, etc. and reference or attach any documentation supporting the initiation of the project, where available.

- A brief summary of the major issue(s) to be addressed by the project – for example:  congestion/LOS/capacity issues, high crash rates, operational issues, geometric or structural deficiencies, legislative program requirements (e.g. GRIP), infrastructure improvements, streetscapes, etc.

- Explanation of the proposed project limits – what conditions exist at the project termini, why should the project terminate at these limits, etc.  Note that Logical Termini are determined as part of the NEPA process.

- Other relevant information regarding the issue(s) the project is intended to address

- Performance goals – in general, what is the major performance goal of the project (e.g. reduce congestion, improve mobility, reduce crashes, correct geometric and/or structural deficiencies, etc.).  Also list any expected secondary benefits the project is expected to provide.

The Project Justification Statement in the Concept Report should not include any information that is not relevant to the issue(s) to be addressed, including demographics/census information, description of possible solutions, etc.

Existing conditions: A brief general description of the project location as it currently is, including lanes, sidewalks, major intersections, structures, and major utilities in project area.

Other projects in the area:  List other projects in the area; include PI numbers and brief description.

MPO:  MPO Name

TIP #: if applicable

TIA Regional Commission:   Regional Commission 	 	If TIA project, list RC Project ID

See Preconstruction Status Report and/or TPro for information

Congressional District(s):   ##

Federal Oversight:	☐ PoDI 	☐ Exempt	☐ State Funded	☐ Other

Projected Traffic:  ADT or AADT		24 HR T:       %

Current Year (20WW):             Open Year (20XX):            	Design Year (20YY):

Traffic Projections Performed by:   GDOT Office or Consulting Firm name

Functional Classification (Mainline):  Functional Classification

Roadway classifications are maintained by Office of Transportation Data

Complete Streets - Bicycle, Pedestrian, and/or Transit Standard Warrants:

Warrants met: 	☐ None         ☐  Bicycle        ☐ Pedestrian	     ☐ Transit

Check all that apply.  Attach summary of Bicycle, Pedestrian, and Transit Warrant Studies or summarize results here.  See Chapter 9 of the GDOT Design Policy Manual for further guidance. Note: If it is not practical to provide appropriate accommodations for GDOT Standard Criteria, Design Variance(s) will be required.

Is this a 3R (Resurfacing, Restoration, & Rehabilitation) Project?	☐ No		☐ Yes

Pavement Evaluation and Recommendations

Initial Pavement Evaluation Summary Report Required?  	☐ No		☐ Yes

Intial Pavement Type Selection Report Required?  	☐ No		☐ Yes

Feasible Pavement Alternatives:  	☐  HMA	☐ PCC	               ☐  HMA & PCC

[HMA = Hot Mix Asphalt; PCC = Portland Cement Concrete] Preliminary Pavement Evaluation Summary and/or Preliminary Pavement Type Selection Reports, if required, should be completed prior to submission of the Concept Report for approval. The Office of Materials and Testing would prepare either or both of these reports upon request.  The pavement report(s) should be attached to the Concept Report. See Chapter 5 of the PDP for further information. Final Pavement Type Selection and pavement design approval occur during the Preliminary Design Phase.

DESIGN AND STRUCTURAL

Description of the proposed project: A general description of the project, including the proposed length, and general location of the project, any city and county limits or proximity thereto. Specific design data (e.g. typical section, design speed, etc.) should be kept to a minimum, since it will be described in a later section.  If an ITS Project, summarize the Concept of Operations briefly. Information on structures should be included in table below.

Major Structures:  (If no major structures on project, N/A and delete table below)

| Structure | Existing | Proposed |
| --- | --- | --- |
| ID # and/or Location | Describe length, typical section, including lane and shoulder widths, etc. of existing structure, and sufficiency rating | Describe proposed length, typical section including lane and shoulder widths, etc. of proposed structure. |
| Retaining walls (not including gravity walls) | Describe current structure | Describe proposed structures |
| Other | Describe current structure | Describe proposed structures |

Mainline Design Features:  Roadway name/identification and Functional Classification

Use additional copies of table below as needed for other major roads, significant sideroads, etc.  Multiple roads with similar characteristics may be combined into a single table as warranted.

| Feature | Existing | Standard* | Proposed |
| --- | --- | --- | --- |
| Typical Section |  |  |  |
| - Number of Lanes |  |  |  |
| - Lane Width(s) |  |  |  |
| - Median Width & Type |  |  |  |
| - Outside Shoulder or Border Area Width |  |  |  |
| - Outside Shoulder Slope |  |  |  |
| - Inside Shoulder Width |  |  |  |
| - Sidewalks |  |  |  |
| - Auxiliary Lanes |  |  |  |
| - Bike Lanes |  |  |  |
| Posted Speed |  |  |  |
| Design Speed |  |  |  |
| Min Horizontal Curve Radius |  |  |  |
| Maximum Superelevation Rate |  |  |  |
| Maximum Grade |  |  |  |
| Access Control |  |  |  |
| Design Vehicle |  |  |  |
| Pavement Type |  |  |  |
| Additional Items as warranted |  |  |  |

*According to current GDOT design policy if applicable

Major Interchanges/Intersections:  List and briefly describe any major interchanges or intersections along project

Lighting required: 		☐ No		☐ Yes

If lighting is included in the project, attach lighting agreements or commitment letters.

Off-site Detours Anticipated:		☐ No		☐ Yes 		☐  Undetermined

If detour is needed, provide a brief justification for detour type selected.  Provide date of Detour Meeting and/or approval date of Detour Report, if available.

Transportation Management Plan [TMP] Required:  ☐ No		☐ Yes

If Yes:	Project classified as:  			☐ Non-Significant	☐ Significant

TMP Components Anticipated: 	☐ TTC		☐ TO		☐  PI

As part of the federal Work Zone Safety and Mobility Rule, all Federal-aid highway projects require a TMP. Projects classified as “Non-Significant” may only require a Temporary Traffic Control (TTC) plan, often covered under Special Provision 150.  Projects classified as “Significant” require a complete TMP and formal TMP report which includes a TTC plan and addresses Transportation Operations (TO) and Public Information (PI) components.  If needed, the formal TMP report would typically be developed during the preliminary plans phase. For more information, see GDOT Policy 5240-1.

Design Exceptions to FHWA/AASHTO controlling criteria anticipated:

| FHWA/AASHTO Controlling Criteria | No | Undeter- mined | Yes | Appvl Date (if applicable) |
| --- | --- | --- | --- | --- |
| - Design Speed | ☐ | ☐ | ☐ |  |
| - Lane Width | ☐ | ☐ | ☐ |  |
| - Shoulder Width | ☐ | ☐ | ☐ |  |
| - Bridge Width | ☐ | ☐ | ☐ |  |
| - Horizontal Alignment | ☐ | ☐ | ☐ |  |
| - Superelevation | ☐ | ☐ | ☐ |  |
| - Vertical Alignment | ☐ | ☐ | ☐ |  |
| - Grade | ☐ | ☐ | ☐ |  |
| - Stopping Sight Distance | ☐ | ☐ | ☐ |  |
| - Cross Slope | ☐ | ☐ | ☐ |  |
| - Vertical Clearance | ☐ | ☐ | ☐ |  |
| - Lateral Offset to Obstruction | ☐ | ☐ | ☐ |  |
| - Bridge Structural Capacity | ☐ | ☐ | ☐ |  |

If any of the above are checked “Yes” or “Undetermined”, please briefly describe the anticipated Design Exception here.  A Design Exception (DE) must be granted for exceeding the FHWA controlling Criteria. Please note that for Full Oversight projects, FHWA generally requires Design Exceptions and Variances to be approved prior to Concept approval.  Attach any approved DE’s to the Concept Report.

Design Variances to GDOT Standard Criteria anticipated:

| GDOT Standard Criteria | Reviewing Office | No | Undeter- mined | Yes | Appvl Date (if applicable) |
| --- | --- | --- | --- | --- | --- |
| - Access Control/Median Openings | DP&S | ☐ | ☐ | ☐ |  |
| - Intersection Sight Distance | DP&S | ☐ | ☐ | ☐ |  |
| - Intersection Skew Angle | DP&S | ☐ | ☐ | ☐ |  |
| - Lateral Offset to Obstruction | DP&S | ☐ | ☐ | ☐ |  |
| - Rumble Strips | DP&S | ☐ | ☐ | ☐ |  |
| - Safety Edge | DP&S | ☐ | ☐ | ☐ |  |
| - Median Usage | DP&S | ☐ | ☐ | ☐ |  |
| - Roundabout Illumination Levels | DP&S | ☐ | ☐ | ☐ |  |
| - Complete Streets | DP&S | ☐ | ☐ | ☐ |  |
| - ADA & PROWAG | DP&S | ☐ | ☐ | ☐ |  |
| - GDOT Construction Standards | DP&S | ☐ | ☐ | ☐ |  |
| - GDOT Drainage Manual | DP&S | ☐ | ☐ | ☐ |  |
| - GDOT Bridge & Structural Manual | Bridges | ☐ | ☐ | ☐ |  |

If any of the above are checked “Yes” or “Undetermined”, please briefly describe the anticipated Design Variance here.  A Design Variance must be granted for exceeding GDOT’s standard criteria.  Attach any approved DV’s to the Concept Report. (NOTE:  If both a Design Exception and Design Variance are indicated for the same deviation, only the Design Exception is required).

VE Study anticipated:  	☐ No		☐ Yes	 	☐  Completed – Date:

If VE Study has been completed, attach VE Implementation letter.

UTILITY AND PROPERTY

Temporary State Route needed:  	☐ No		☐ Yes		☐ Undetermined

By State Code (32-5-2), federal and state transportation funds cannot be used to procure rights-of-way for off-system routes. To utilize federal or state funds for off-system routes, a temporary State Route number must be assigned to the county road or city street before purchasing right-of-way. Contact the System Highway Coordinator in the Office of Transportation Data (HighwaySystemsAdministrator@dot.ga.gov) if more information is needed. If a temporary State Route is needed, please identify the roadway and approximate limits.

Railroad Involvement: If there are any railroads in the project vicinity which may be affected directly or indirectly by the project, list them here. Discuss ownership and future use of the Railroad (e.g. proposed new rail lines, freight or passenger rail, number of trains per day, etc.).  Also list whether any RR

coordination is needed. A cost estimate for RR coordination should be attached, if applicable. Consult State Railroad Coordinator in Office of Utilities for RR coordination requirements.

Utility Involvements: List any identified utilities which may be impacted by project, including type and owner.

SUE Required:  	☐ No		☐ Yes		☐ Undetermined

Note:  By policy, SUE is required for all projects with a Commissioner approved Public Interest Determination Recommendation.

Public Interest Determination Policy and Procedure recommended?	 ☐ No		☐ Yes

See Policy and Procedures Subject Nos. 6863-12 and 3E-1 for guidance.  If yes, describe the Concept Team’s findings and recommendations. Attach Utility Risk Management Plan with Risk Acceptance or Risk Avoidance recommendations if applicable.

Right-of-Way (ROW): 	Existing width:        ft.		Proposed width:        ft.

Refer to Chapter 3 of GDOT’s Design Policy Manual for guidance.

Required Right-of-Way anticipated:	☐ None    	☐ Yes		☐ Undetermined

Easements anticipated: 	☐ None   ☐ Temporary   ☐ Permanent   ☐ Utility   ☐ Other

Check all easement types that apply.

| Anticipated total number of impacted parcels: |  |  |
| --- | --- | --- |
| Displacements anticipated: | Businesses: |  |
|  | Residences: |  |
|  | Other: |  |
| Total Displacements: |  |  |

Location and Design approval:	☐ Not Required		☐ Required

Note:  Location and Design approval is needed for all projects where ROW or easements are to be acquired.

Impacts to USACE property anticipated?	☐ No	☐ Yes	☐ Undetermined

Under 33 USC 408, if additional property rights from USACE property are anticipated, a 408 Decision  may be required. The Project Manager should contact the State Design Policy Engineer when a potential impact to USACE property is identified. The State Design Policy Engineer will assess the potential impact(s) and determine if further coordination is needed. Obtaining a 408 Decision may require considerable coordination and effort.

ROUNDABOUTS See GDOT Design Policy Manual - Chapter 8 for further guidance.  Delete this section if project does not include a roundabout.

Roundabout Lighting Agreement/Commitment Letter received:  ☐ No	☐ Yes

Agreement or commitment letter should be attached

Roundabout Planning Level Assessment:  Briefly explain the findings of the Planning Level Assessment and attach Planning Level Assessment to Concept Report. Required for all projects containing roundabouts where a Roundabout Feasibility Study has not been prepared.  This includes linear projects where a roundabout is proposed.

Roundabout Feasibility Study:  Summarize the findings of the Roundabout Feasibility Study and attach Roundabout Feasibility Study to Concept Report.  In most cases, the components of a feasibility study can be directly incorporated into the body of the Concept Report and no separate feasibility study prepared.  Not required during concept for linear projects where roundabout(s) are proposed.

Roundabout Peer Review Required:   ☐ No	☐ Yes		☐  Completed – Date:

Attach Peer Review Report and responses to all report comments not incorporated into the design.

CONTEXT SENSITIVE SOLUTIONS

Issues of Concern:   Briefly list potential project impacts that have been identified which may require Context Sensitive Solutions.  Refer to GDOT’s Context Sensitive Design Online Manual and AASHTO’s Guide for Achieving Flexibility in Highway Design.

Context Sensitive Solutions Proposed:  Describe how the Issues of Concern listed above are to be addressed by the project.

ENVIRONMENTAL & PERMITS

Anticipated Environmental Document:

GEPA:  ☐ 		NEPA:   ☐ CE		☐ EA/FONSI		☐ EIS

MS4 Permit Compliance – Is the project located in a MS4 area?	☐ No		☐ Yes

For projects within a designated MS4 (Municipal Separate Storm Sewer Systems) area, at a minimum, the conceptual project cost estimate (PE, ROW, UTIL, CST, ENV MIT, etc.) shall include preliminary, estimated costs related to the impacts that MS4 post construction structures may have. When sufficient topography and drainage information is available, a rudimentary sizing of structures should be performed to better define any additional right of way requirements and develop an updated overall project cost estimate.  Information on Georgia’s MS4 Permit can be found at Georgia EPD’s Website.

For more information regarding GDOT’s MS4 permit, please contact the Hydraulic Studies Group in the Office of Design Policy & Support.

Environmental Permits/Variances/Commitments/Coordination anticipated:  List all anticipated permits, variances, commitments, and coordination needed –Section 404, TVA, Water Quality, etc.

| Permit/ Variance/ Commitment/ Coordination Anticipated | No | Yes | Remarks |
| --- | --- | --- | --- |
| - U.S. Coast Guard Permit | ☐ | ☐ |  |
| - Forest Service/Corps Land | ☐ | ☐ |  |
| - CWA Section 404 Permit | ☐ | ☐ |  |
| - 33 USC 408 Decision | ☐ | ☐ |  |
| - Tennessee Valley Authority Permit | ☐ | ☐ |  |
| - Buffer Variance |  | ☐ |  | ☐ |  |  |  |
| - Coastal Zone Management Coordination | ☐ | ☐ |  |
| - NPDES | ☐ | ☐ |  |
| - FEMA | ☐ | ☐ |  |
| - Cemetery Permit | ☐ | ☐ |  |
| - Other Permits | ☐ | ☐ |  |
| - Other Commitments | ☐ | ☐ |  |
| - Other Coordination | ☐ | ☐ |  |

Use this area below the table for more details on Permits/Variances/Commitments/Coordination Anticipated as needed.

Is a PAR required?	☐ No		☐ Yes		☐ Completed – Date:

If PAR has been completed, attach PAR report.  Note:  A PAR, if required, should normally be completed prior to Concept Report submission.

Environmental Comments and Information:

NEPA/GEPA:  List status of environmental document and comment on any significant NEPA/GEPA issues and/or risks present including 4f resources.

Ecology:  List level of study performed, if any protected species or habitats may be present, seasonal survey requirements, and any other significant issues that should be considered throughout project development.

History:  List possible effects to potential or known historic resources, if additional surveys are required, if SHPO concurrence is required or has been received, and any other significant issues that should be considered throughout project development.

Archeology:  List any cemeteries or other publicly documented archeological resources present, possible effects to archeological resources, if additional surveys be required, if SHPO concurrence is required or has been received, and any other significant issues that should be considered throughout project development.

Air Quality:

Is the project located in a PM 2.5 Non-attainment area?	☐ No		☐ Yes

Is the project located in an Ozone Non-attainment area?	☐ No		☐ Yes

Carbon Monoxide hotspot analysis:	☐ Required	   ☐ Not Required 	☐ TBD

If yes to either PM 2.5 or Ozone Non-attainment, provide a comparison between the proposed project concept and the conforming plan’s model description. Include such features as project limits, number of through lanes, proposed open to traffic year, etc.  If project is exempt from conforming plan, explain why. If the project corridor contains a traffic signal, the design year traffic volumes exceed 10,000 vpd and the level of service is D, E or F, a CO hotspot analysis is required.

Noise Effects:  List level of noise studies required, modeling requirements, mitigation measures needed, etc.

Public Involvement:  List level of Public Outreach expected including citizen committees, Public Information meetings, Public Hearings, Detour Meetings, etc.; also include any additional public outreach needed. For significant meetings previously completed, list dates, types of meetings, and attach meeting summaries or minutes.

Major stakeholders:  Identify major stakeholders in project (e.g. traveling public, business associations, etc.).

CONSTRUCTION

Issues potentially affecting constructability/construction schedule:  Summarize any known issues which may affect the construction of the project (e.g. staging/detour issues, seasonal construction requirements, very high traffic volumes requiring off-hour construction, etc.

Early Completion Incentives recommended for consideration:  ☐ No		☐ Yes

Early Completion Incentives is a method of providing the contractor with an incentive to expedite the completion of construction.  Appropriate projects are those which address severe congestion – to provide an early benefit - or where construction must be completed by a fixed date.   Incentives should only be considered where recommended by the Office of Construction.  If incentives for early completion are recommended for consideration, include brief explanation of major reasons why and include estimate of RUC (Road User Costs). A benefit-to-cost ratio calculation may be required.

COORDINATION, ACTIVITIES, RESPONSIBILITIES, AND COSTS

Initial Concept Meeting:  (if applicable) - Provide date of ICM and brief summary.  Attach minutes if available.

Concept Meeting:  Provide date of CM and brief summary.  Attach minutes.

Other coordination to date:  Attach any pertinent documentation of other meetings/coordination.

| Project Activity | Party Responsible for Performing Task(s) |
| --- | --- |
| Concept Development | GDOT Office, Consulting firm, local government, etc. |
| Design |  |
| Right-of-Way Acquisition |  |
| Utility Coordination (Preconstruction) |  |
| Utility Relocation (Construction) |  |
| Letting to Contract |  |
| Construction Supervision |  |
| Providing Material Pits |  |
| Providing Detours |  |
| Environmental Studies, Documents, & Permits |  |
| Environmental Mitigation |  |
| Construction Inspection & Materials Testing |  |

Project Cost Estimate Summary and Funding Responsibilities:  Add additional rows as necessary; Attach current cost estimates to report.

|  | Breakdown of PE | ROW | Reimbursable Utility | CST* | Environmental Mitigation | Total Cost |
| --- | --- | --- | --- | --- | --- | --- |
| Funded By |  |  |  |  |  |  |
| $ Amount |  |  |  |  |  |  |
| Date of Estimate |  |  |  |  |  |  |

*CST Cost includes: Construction, Engineering and Inspection, Contingencies and Liquid AC Cost Adjustment.

ALTERNATIVES DISCUSSION

Alternative selection:  Compare and contrast the various alternatives studied in summary and reason(s) why each alternative was or was not selected.   Discussion should include no-build and preferred alternatives, and should compare various factors such as total cost, environmental and social impacts, time requirements, PE requirements, etc. as appropriate to the decision process.  Please use the following format:

| Preferred Alternative: description |  |  |  |
| --- | --- | --- | --- |
| Estimated Property Impacts: |  | Estimated Total Cost: |  |
| Estimated ROW Cost: |  | Estimated CST Time: |  |
| Rationale: Reason(s) why this alternative was or was not selected (cost, property impacts, environmental impacts, etc.). Preferred build alternative should meet goals outlined in Project Justification. |  |  |  |

| No-Build Alternative: description |  |  |  |
| --- | --- | --- | --- |
| Estimated Property Impacts: |  | Estimated Total Cost: |  |
| Estimated ROW Cost: |  | Estimated CST Time: |  |
| Rationale: Reason(s) why this alternative was or was not selected (cost, property impacts, environmental impacts, etc.). |  |  |  |

| Alternative 1: description |  |  |  |
| --- | --- | --- | --- |
| Estimated Property Impacts: |  | Estimated Total Cost: |  |
| Estimated ROW Cost: |  | Estimated CST Time: |  |
| Rationale: Reason(s) why this alternative was or was not selected (cost, property impacts, environmental impacts, etc.). |  |  |  |

Continue with Alternative 2, 3, etc. as appropriate.

Comments:  Add further comments as appropriate.

LIST OF ATTACHMENTS/SUPPORTING DATA (List supporting data in attached order)

- Concept Layout

- Typical sections

- Detailed Cost Estimates:

  - Construction including Engineering and Inspection and Contingencies

  - Completed Liquid AC Cost Adjustment forms

  - Right-of-Way

  - Utilities

e.   Environmental Mitigation

- Crash summaries

- Traffic diagrams

- Capacity analysis summary (tabular format)

- Summary of TE Study and/or Signal Warrant Analysis

- Roundabout Data (if applicable – see GDOT Design Policy Manual)

  - Planning level assessment

  - Roundabout feasibility study

  - Lighting agreement or commitment letter

  - Peer Review and responses

- S I & A Report(s) (Bridge/Structural Inventory Report(s) - If applicable)

- Concept Level Hydrology Study for MS4 Permit (if applicable)

- Pavement studies (e.g. Preliminary Pavement Type Selection Report, etc.)

- Utility Risk Management Plan (If available - Derived from the Public Interest Determination Policy and Procedure)

- Conforming plan’s network schematics showing thru lanes.  (Note:  This  attachment  is required for non-attainment areas only)

- Minutes of Concept meetings

- Minutes of any meetings that shows support or objection to the concept (e.g. PIOH, PHOH, Detour Meeting, Town Hall Meeting, etc.)

- PFA’s and/or SAA’s

- Other items referred to in the body of the report as applicable

APPROVALS

| Concur: |  |  |  |
| --- | --- | --- | --- |
|  | Director of Engineering |  |  |
| Approve: | Include this signature line for PoDI Projects Only |  |  |
|  | Division Administrator, FHWA |  | Date |
| Approve: |  |  |  |
|  | Chief Engineer |  | Date |

Intentionally Left Blank