# ECOLOGY post-construction stormwater report

PI Number: 					                Submittal Date:

Project Name: 					    Company

County: 					                Baseline Let Date:

District: 				   		    Contact Name:

Contact Email:                          _                                 Contact Phone:

Purpose of Report: __________________________________________

____________________________________________________________

Pollutant(s) of Concern: _____________________________________

Project Specific Treatment Targets: __________________________ ____________________________________________________________ ____________________________________________________________

Net Length of Project: ________________________miles

Existing Typical Section: Urban ☐   Rural ☐  No. Lanes_______

Proposed Typical Section: Urban ☐   Rural ☐  No. Lanes_____

Project Type:________________________________________________

PE Seal, Signature, & Date

Note: Not required if report is prepared by GDOT

# Ecology post-construction stormwater guidance

The template is meant to serve as a guide and will not be all encompassing of all projects.  It is intended to be flexible and if changes to the template are needed for a specific project, the engineer of record should exercise their judgement when making changes to the format.

Please see the GDOT Drainage Manual for definitions of acronyms.

Area of Potential Effect

This report template is intended to be used to document the feasibility analysis of post-construction water quality treatment for ecology purposes outside of MS4 areas per agency coordination or other obligations associated with ecological resources of concern. Only those drainage areas directly associated with the ecological resources of concern should be included in reports generated from this template.

For traditional delivery projects, the ecologist will provide the information for the first two columns of Attachment A and the designers will fill out the rest of the report and submit to EnvBMP@dot.ga.gov, copying the consultant and GDOT ecologists, at least 22 weeks prior to PFPR. The report will be reviewed ODPS and revised by the designer until the report is found to be acceptable. Ideally, the report will be found to be acceptable at least 18 weeks prior to PFPR. When submitting, also include a link to the current construction plans. The submittal may be done via the PM through ProjectWise, by WeTransfer, or by a similar file transfer service.

For Innovative Delivery projects, the ecologist will provide the information for the first two columns of Attachment A and the pre-let GEC will fill out the rest of the report and submit to EnvBMP@dot.ga.gov, copying the consultant and GDOT ecologists in time for there to be at least one round of comments and resubmittal. The report will be reviewed ODPS and revised by the pre-let GEC until the report is found to be acceptable. Then the EPCSR will be included in the Ecology AOER. After the developer receives NTP 2, the post-let GEC will review the EPCSR and revise if needed. The post-let GEC will submit to EnvBMP@dot.ga.gov in time for there to be at least one round of comments and resubmittal. The report will be reviewed by ODPS and revised by the post-let GEC until the report is found to be acceptable. Then the final EPCSR and lockdown plans will be sent to NEPA for re-evaluation and Ecology AOER addenda prior to NTP 3.

Note: Since this template is only for projects outside of MS4 areas, when agency coordination with ecologists indicates that water quality treatment needs to be assessed on a project inside an MS4 area, the MS4 Post Construction Stormwater Report Template will be used. When submitting the MS4 Post Construction Stormwater Report, also include Appendix F from this template. In this instance, analyze drainage areas in the area of potential effect for the ecology resource according to the practicability criteria within this template to determine practicability of individual post-construction BMPs. The MS4 PCSR will still require infeasibility analysis, however the deciding factors are likely to be the practicability criteria for ecology purposes.

Ecology Post Construction Stormwater Report Addendum Process

An addendum should be submitted if any of the following changes occur after the Ecology PCSR has been submitted to the applicable regulatory agency and considered final. The addendum should be submitted to EnvBMP@dot.ga.gov, copying the consultant and GDOT ecologists, 4 weeks prior to lockdown plans. The ecologist and ODPS should discuss to assess if the changes are certain enough to notify the applicable agency. Include the following in the submittal of the addendum:

- An outfall not previously considered has been identified.

- The treatment practicability determination has changed for an outfall (from practicable to not practicable, or vice-versa).

- The type of post-construction BMP recommended at an outfall has changed.

- A protected species or habitat for a protected species for which water quality is a primary concern was found in an area of the project where water quality treatment had not previously been evaluated.

- Project activities have changed such that coordination under the Fish and Wildlife Coordination Act (FWCA) is now required, or needs to be re-opened to address additional impacts or additional resources.

The addendum needs to address only the drainage basins where such changes have occurred. Submit a link to the current construction plans as well as the following as a draft in PDF format for review:

- Cover letter outlining the changes

- Revised cover page

- Revised Attachment A

GDOT POST-CONSTRUCTION SUMMARY

Attachment A is the GDOT Post-Construction BMP Summary.  The ecologist will provide the information for the first two columns of this form and the rest should be completed by the designer. The fully filled out Attachment A should be submitted with all ecology stormwater reports. The purpose of the documentation is to assist with the plan review process.

PRACTICABLITITY OF POST-CONSTRUCTION BMPs

There are certain instances where it may not be practicable to implement post-construction BMPs. A written explanation supporting the application of any of the following criteria shall be provided in Attachment B.

Criteria for when water quality treatment may not be practicable include the following:

- The implementation of post-construction BMPs would require changing the existing roadway alignment resulting in a safety concern.

- The installation of post-construction BMPs would require in-line structures and/or fill in a stream.

- The installation of post-construction BMPs would impact stream buffers or wetlands where impacts would otherwise be avoided.

- Where stormwater discharges from the project site are designed to exit the right-of-way as sheet flow (non-point source discharges).  The designer should determine if this is possible by visiting the site, and providing a written explanation with supporting evidence for this drainage area. (Project-specific agreement with regulatory agencies may supersede this criterion.)

- Stormwater runoff to be treated does not apply to flows that originate outside of project right-of-way or diverted flows from undisturbed areas.

- Implementation of BMPs would cause loss of habitat for or adversely affect protected species. (Project-specific agreement with regulatory agencies may overturn this criterion.)

- Implementation of BMPs would cause significant damage to a cultural or community resource such as an historical site, archeological site, cemetery, park, wildlife refuge, library, or school facilities. (Project-specific agreement with regulatory agencies may supersede this criterion.)

- Implementation of BMPs would result in the displacement of a residence or business which would not have otherwise been displaced.

- Site limitations such as shallow bedrock, contaminated soils, high groundwater, utilities, or underground facilities would prohibit the implementation of post-construction BMPs. (Project-specific agreement due to specific species may overturn this criterion.)

- Soil infiltration capacity is too low or site conditions prevent infiltration of the design volume (applies only to infiltration BMPs).

- Site does not allow for gravity flow to the appropriate BMP.

Issues to consider include FEMA regulations and clear zone requirements. Additionally, consider other state or federal laws, regulations, and polices. If the BMP would be placed in a FEMA zone or clear zone, contact OES and ODPS for coordination.

For outfalls along linear roadway projects whereby the net impervious surface area within that outfall’s drainage area has been reduced, remains the same as pre-developed conditions, or increases less than 0.1acre, post-construction stormwater requirements do not apply. (Project-specific agreement may overturn this criterion.)

POST-CONSTRUCTION BMP SELECTION

Current GDOT policy allows the following eleven BMPs for post-construction stormwater management:

|  |  | Treatment Parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMP |  | RR% | WQV | TSS Removal | TP | TN | Fecal Coliform | Metals | Temperature Reduction |
| Filter Strip |  | 25% | Yes | 60% | 20% | 20% | NA | 40% | Yes |
| Grass Channel | A & B HSG | 25% | Yes | 50% | 25% | 20% | NA | 30% | Yes |
|  | C & D HSG | 10% |  |  |  |  |  |  |  |
| Enhanced Dry Swale | Open Underdrain | 50% | Yes | 80% | 50% | 50% | NA | 40% | Yes |
|  | Capped underdrain | 100% |  | 100% | 100% | 100% | NA | 100% | Yes |
| Enhanced Wet Swale |  | 0% | Yes | 80% | 25% | 40% | NA | 20% | NA |
| Infiltration Trench |  | 100% | No | 100% | 100% | 100% | 100% | 100% | Yes |
| Sand Filter |  | 0% | Yes | 80% | 50% | 25% | 40% | 50% | Yes |
| Dry Detention Basin |  | 0% | Yes | 60% | 10% | 30% | NA | 50% | NA |
| Wet Detention Pond |  | 0% | Yes | 80% | 50% | 30% | 70% | 50% | NA |
| Stormwater Wetland – Level 1 |  | 0% | Yes | 80% | 40% | 30% | 70% | 50% | NA |
| Stormwater Wetland – Level 2 |  | 0% | Yes | 85% | 75% | 55% | 85% | 60% | NA |
| Bioslope | A & B HSG | 50% | Yes | 85% | 60% | 25% | 60% | 75% | Yes |
|  | C & D HSG | 25% |  |  |  |  |  |  |  |
| Bioretention Area | Open underdrain | 50% | Yes | 85% | 80% | 60% | 90% | 95% | Yes |
|  | Upturned underdrain | 75% |  |  |  |  |  |  |  |
|  | Capped underdrain | 100% |  | 100% | 100% | 100% | 100% | 100% | Yes |
| Open Graded Friction Course (OGFC) |  | 0% | Yes | 80% | NA | NA | NA | NA | NA |

Certain BMPs may be used in a “treatment train.” If used in a treatment train, the TSS removal for the treatment train would be calculated by taking the percentage TSS removal of the first BMP in the treatment train and adding the product of the remaining TSS (the percentage TSS not removed by the first BMP) and the TSS removal rate of the second BMP in the treatment train. For example, if a filter strip and a grass channel were used together, the total TSS removal for the treatment train would be calculated as 60% (for the filter strip) + (40% remaining TSS)* 50% (for the grass channel), which would result in a total TSS removal of 80%.

BMPs with the lowest total cost should be evaluated first. The lowest total cost is defined as the lowest cost BMP with the lowest long-term maintenance cost that will provide the requested treatment for the drainage area. Note that the long-term maintenance cost of a BMP should be used only in determining the order of appropriate BMP evaluations, and should not be used to justify why a BMP is not appropriate for a drainage basin or as a criteria for not being practicable. If the first BMP evaluated is determined inappropriate or infeasible, successive BMPs adequate to provide the requested treatment should be evaluated in order of increasing cost until an appropriate BMP is selected or all of the BMPs are eliminated.

The details of the BMP evaluation process shall be provided in Attachment B. All supporting information (figures, calculations, and other backup data) should be included in Appendices A – G. To facilitate completion of Attachment B, the template provides user instructions (red text; remove from final document), as well as example verbiage (black text, with blue text to be replaced by project-specific information).

Attachment A

GDOT Post-Construction BMP Summary

| Planning Considerations |  |  |  |  |  |  | Location and Identification |  |  | Maintenance Responsibility (If the BMP is within GDOT ROW then GDOT will maintain the BMP. If the BMP is on local ROW, coordinate with the locals.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water Resource (Filled Out By Ecologist) | Purpose for Analysis of Water Resource | Impairment (if applicable) | Outfall Label (Drainage Basin Label) | Evaluated BMP(s) | Evaluation of Practicability | Practicable Selected BMP(s) (if applicable) | Station (Begin - End) | Offset (Left/ Right) | Plan Sheet |  |

Note 1: If a BMP is identified as not practicable include the criteria number (as listed in the Ecology Post-Construction Stormwater Report Guidance section) and provide supporting evidence.

Note 2: The designer should assess any need for attenuation of peak flows separately from this report (unless project-specific agreement indicates otherwise).

ATTACHMENT B

Ecology Post-Construction Stormwater BMP Evaluation and Supporting Documentation

NOTE:

In general, GDOT is looking for concise summaries for each basin evaluation. Discussion of which BMPs were practicable, which BMPs were not practicable, why the BMPs were selected, and documentation supporting the determination with calculations and/or drawings should be included in this attachment.  Please refer to the remainder of this template for further guidance.

Project Description

Provide the most current project description available for the project. It should include a general overview of the project and unique site conditions.

GDOT proposes to [widen I-85 to add one managed lane in each direction from just north of Old Peachtree Road to Hamilton Mill Road. To minimize impacts to traffic, two new auxiliary lanes will be constructed where required. The added managed lanes will be tolled to create reliable travel time savings through the use of variable priced tolling to manage lane volume while maintaining a minimum average speed.]

This project is divided into [3] proposed drainage basins within the Area of Potential Effect. Refer to Appendix A for drainage basin delineations.

Maintenance Discussion

The Design Phase Leader should discuss the maintenance accessibility and responsibility with GDOT Maintenance/District Maintenance for a selected BMP including the consideration and use of local municipal maintenance forces and required agreements.  This section is used to document this discussion. Ensure Attachment A accurately represents the outcome of the discussion.

Basin Evaluation

This section is used to discuss the evaluation process and conclusions for each drainage basin. One example basin is shown:

Drainage Basin 1 (Note: change basin name/number to correspond with Attachment A)

Include a brief description of the drainage basin location and where it discharges.

Drainage Basin 1 is located along [road name] between station [0+00] and station [0+00]. This drainage basin discharges [directly into an existing drop inlet and 15-inch reinforced concrete pipe that discharges outside of the right-of-way].

Physical Parameters of Drainage Basin 1

Provide the pre- and post-development impervious and other land use of the basin used in the calculation of the flows, using the NRCS TR-55 Method. Include the hydrologic soil group and the composite curve number (CN) in the summary tables.

| Drainage Basin 1 (Pre) | Area (ac) | CN |
| --- | --- | --- |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 0.25 | 61 |
| Open space - Good condition (grass cover > 75%) (Soil Group C) | 0.13 | 74 |
| Impervious | 1.35 | 98 |
| Woods - Good condition (Soil Group B) | 0.42 | 55 |
| Total | 2.15 | 84 |

| Drainage Basin 1 (Post) | Area (ac) | CN |
| --- | --- | --- |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 0.02 | 61 |
| Impervious | 1.71 | 98 |
| Woods - Good condition (Soil Group B) | 0.42 | 55 |
| Total | 2.15 | 89 |

Water Quality Volume

| Total Drainage Area (ac) | 2.15 |
| --- | --- |
| Pre-Developed Impervious Area (ac) | 1.35 |
| Post-Developed Impervious Area (ac) | 1.71 |
| Pre-Developed % Impervious | 62.79 |
| Post-Developed % Impervious | 79.53 |
| Runoff Coefficient (Rv) | 0.151 |
| Target RRv (ft3) | 1,178 |
| Target WQv (ft3) | 1,411 |

Supporting stormwater runoff quality (water quality volume) calculations are included in Appendix D.

Best Management Assessment (BMP) Selection

Assess each of GDOT pre-approved BMPs for suitability as an appropriate BMP for the basin. Refer to Table 10.5-1 BMP Screening Criteria in the Drainage Manual for additional guidance. Selection criteria for selecting an appropriate BMP can include: treatment requested; cost (capital); maintenance restrictions; safety (motorist, GDOT maintenance staff, and general public); and site constraints (available right-of-way, soils, bedrock and water table, topography, setback requirements, and environmentally or socially sensitive areas). Start the evaluation with any infiltration BMPs included on the list of appropriate BMPs. Assess suitability of proposed topography to meet design criteria of BMPs. Altering proposed topography for the purposes of installing a BMP should also be considered (ex. regrading ditch or side slope) at this point, with the resulting effects to be assessed during the practicability assessment stage (i.e. additional earthwork costs, right-of-way impacts, etc.).

Filter Strip – The typical section for this section of the project is an interstate section with guardrail or side barriers. No sheet flow from impervious areas is present within the drainage limits. Therefore, the filter strip is not an appropriate BMP for this basin.

Grass Channel – The grass channel provides only 50% TSS treatment of the WQv. However, since this analysis is for ecology purposes, the grass channel is appropriate.

Infiltration Trench – The infiltration trench is an appropriate BMP for this basin. However, the enhanced swale is more appropriate so the infiltration trench will not be considered for this basin.

Sand Filter – The sand filter is an appropriate BMP for this basin.

Bioretention Area – The bioretention area is an appropriate BMP for this basin.

Dry Detention Basin – The dry detention basin is an appropriate BMP for this basin.

Wet Detention Pond – The drainage area is less than 10 acres. Therefore, the wet detention pond is not an appropriate BMP for this basin.

Stormwater Wetland – The drainage basin is less than 5 acres. Therefore, the stormwater wetland is not an appropriate BMP for this basin.

Bioslope – The typical section for this section of the project is an interstate section with guardrail or side barriers. No sheet flow from impervious areas is present within the drainage limits. Therefore, the bioslope is not an appropriate BMP for this basin.

Enhanced Dry Swale – The enhanced dry swale is an appropriate BMP for this basin.

Enhanced Wet Swale – A permanent pool cannot be maintained. Therefore, the enhanced wet swale is not an appropriate BMP for this basin.

Open Graded Friction Course – OGFC is not present throughout the project.

Practicability

For each BMP that was deemed appropriate for the drainage basin based on meeting design constraints and requirements, assess the practicability per GDOT’s fourteen criteria. Provide BMP sizing calculations, displays, etc. in the appendices of the report to justify the claim(s).

Enhanced Dry Swale – The enhanced dry swale was first evaluated. Any attempt to construct the enhanced dry swale over the required length will result in impacts to a wetland buffer which would not have been impacted otherwise by the project. This BMP is not practicable under criteria #3.

Bioretention Area – The bioretention area with a capped underdrain was first evaluated. It could not be constructed without using cast in place walls. The BMP is not practicable under criteria #11.

Sand Filter – The sand filter and sedimentation basin would be constructed using cast in place walls. The BMP is not practicable under criteria #11.

Dry Detention Basin – The dry detention basin could not be constructed without using cast in place walls. The BMP is not practicable under criteria #11.

Grass Channel- The Grass Channel was evaluated last as it does not provide 80% TSS removal. However, since 80% TSS removal is not practicable, Grass Channel was evaluated and found to be practicable.

Drainage Basin 1 was found to be not practicable for 80% TSS removal. However, the Grass Channel was practicable to provide 50% TSS removal for the net new impervious area.

Appendix A –Drainage Basin Maps

Include maps showing the pre- and post-development drainage basins. Be sure to show and label the water resources of concern.

Appendix B – Soils Map

Include soil map(s) showing the hydrologic soil group in each drainage basin.

Appendix C – Stormwater Runoff Quality Calculations

Include water quality volume and runoff reduction volume calculations for those drainage basins that are practicable to treat.

Appendix D – Practicability Documentation

Provide backup documentation supporting each criterion claimed.

Appendix E – BMP Design Calculations

Provide the design calculations for practicable BMPs.

Appendix F – Agency Coordination for Treatment Targets

Provide the original coordination with the agency for treatment commitments. The consultant and GDOT ecologists, ODPS, and the agency should be included in this coordination.

List initial project specific treatment parameters (Area of Potential Effect, pollutants of concern, protected species, percent removal targets, net impervious treatment versus total impervious treatment, volume calculation parameters (WQv, RRv, etc). Send the list to the agency and receive written or email concurrence on the agreed upon initial treatment parameters.

During this coordination, decide if a project-specific agreement will overturn any practicability criterion. Record the results in this Appendix.

If the commitments have been revised, include correspondence with the agency which advises of updated commitments.

Appendix G – Cost Calculations

Provide a cost estimate for each recommended BMP (per each) and cost estimate for ROW acquisition for each recommended BMP. Please note this is not a practicability criterion for ecology stormwater but it is a documentation step. Please use the example tables below. If using the tables below, please fill out a separate table for each recommended BMP as ROW costs will be different for different BMPs. Please assume $5,000 for landscaping on BMPs which require landscaping (bioretention basins, wet detention basins, wet swales, stormwater wetlands).

|  | Bioslope # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Bioslope | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $750 | $15,000 |
|  |  |  | Total |  |

|  | Enhanced Dry Swale # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Enhanced Dry Swale | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $1,000 | $20,000 |
|  |  |  | Total |  |

|  | Enhanced Wet Swale # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Enhanced Wet Swale | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $1,000 | $20,000 |
| Landscaping | Lump Sum | 1 | $5,000 | $5,000 |
|  |  |  | Total |  |

|  | Bioretention Basin # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Bioretention Basin | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $3,000 | $60,000 |
| Landscaping | Lump Sum | 1 | $5,000 | $5,000 |
|  |  |  | Total |  |

|  | Dry Detention Basin # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Dry Detention | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $2,000 | $40,000 |
|  |  |  | Total |  |

|  | Wet Detention Basin # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Wet Detention | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $3,000 | $60,000 |
| Landscaping | Lump Sum | 1 | $5,000 | $5,000 |
|  |  |  | Total |  |

|  | Sand Filter # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Sand Filter | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $3,000 | $60,000 |
|  |  |  | Total |  |

|  | Stormwater Wetland # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Stormwater Wetland | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $5,000 | $100,000 |
| Landscaping | Lump Sum | 1 | $5,000 | $5,000 |
|  |  |  | Total |  |

|  | Infiltration Trench # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Infiltration Trench | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $7,000 | $140,000 |
|  |  |  | Total |  |

Filter strips and grass channels do not have associated pay items and are assumed to cause no extra cost unless additional grading and/or ROW is required (if this is the case, provide a cost estimate for the additional grading and/or ROW).

Cost calculations are not needed for OGFC given that OGFC is only used if the project already needed OGFC.