Revised February 9, 2017

Note to the Engineer:  Use this special provision for contractor provided Pile Driving Analyzer (PDA) for non-LRFD projects.  The date in the top right hand corner is a revision date and should not be changed. In addition, delete this note when you prepare and print this special provision.

DEPARTMENT OF TRANSPORTATION

STATE OF GEORGIA

SPECIAL PROVISION

PROJECT No. 00000-0000-00(000), County

P.I. No. 0000000

SECTION 523 - DYNAMIC PILE TESTING

523.1 General Description

The work consists of performing dynamic pile testing using the Pile Driving Analyzer (PDA) to monitor the driving of piles with accelerometer and strain gauges attached to the piles. Piles to be dynamically tested will be identified in the Special Provision or on the Plans. Prior to pile driving, the Engineer will determine production or test piles to be dynamically tested. Perform the dynamic pile testing in accordance with ASTM D4945-12.

Take dynamic measurements during driving of any required piles. Drive the pile as shown in the Special Provisions or on the Plans.

523.2	Materials

Furnish measuring instruments for dynamic pile testing. Attach instruments near the top of the piles with bolts placed in drilled holes. Furnish materials, labor and equipment necessary for installation of the instruments.

523.3	Construction Requirements

Measure wave speed prior to driving piles. Wave speed measurements will not be required for Steel H piles or metal shell piles. When wave speed measurements are performed, place the piles in a horizontal position not in contact with other piles.

Perform dynamic pile testing during driving. Modify the driving to reduce the stress and/or eliminate the damage, should the recommended stress level be exceeded or if damage occurs (determined visually or as indicated by the instrumentation).

Do not exceed the following maximum driving stresses, as determined by the dynamic pile testing:

- For Steel piles:

0.9 Fy, where Fy = Yield strength of steel

- For Prestressed Concrete Piles:

Compression:

Sapc = 0.7 f ′c − 0.75 f pe

Tension:

Sapt = 6.5 (f ′c) 0.5+1.05 f pe		(for piles less than 50 feet long)

Sapt = 3.25 (f ′c) 0.5 +1.05 f pe	(for piles 50 feet long and greater)

where;

Sapc= maximum allowed pile compressive stress, psi

Sapt= maximum allowed pile tensile stress, psi

f ′c= specified minimum 28-day compressive strength of concrete, psi

f pe= effective prestress in concrete, psi, (after all losses) at the time of driving taken as

0.8 times the initial prestress force

- For timber piles:

0.9 Ft, where Ft = Ultimate strength of the wood parallel to the grain

Re-drive friction piles that do not obtain bearing after a freeze period of a minimum of 24 hours or for a period designated on the Plans, whichever is longer. Reset the gauges if required. Re-strike the pile with a warm hammer until a maximum penetration of 3 inches (76 mm) or 40 blows is reached, whichever occurs first. The Engineer may modify the Pile Driving Objective based on the results of the PDA work.

Provide two weeks’ notice prior to the driving of designated piles and cooperate with the Engineer in connection with the performance of Dynamic Pile Testing.

Provide a complete report consisting of but not limited to PDA field monitoring data, results of CAPWAP computer analyses, and recommendations such as pile lengths, hammer fuel setting, and valid driving criteria.  Valid driving criteria is defined as having the required hammer having a hammer set greater than 3 blows per inch and less than 10 blows per inch at the driving resistance for that pile. Submit the report electronically in PDF format and the electronic data files of the PDA analysis and CAPWAP to the Geotechnical Bureau and allow seven (7) calendar days for review and approval before proceeding with driving production piles.

523.4	Measurement

The Dynamic Pile Tests performed in accordance with these Specifications will be counted separately for payment. (Refer to plans summary sheet for the required amount of PDA testing.)

523.5	Payment

The Dynamic Pile Test completed and accepted will be paid for at the Contract unit Price. This payment will be full compensation for all costs of complying with this specification, including incidentals, additional work, and any delays incurred in conjunction therewith.

Payment will be made under:

Item No. 523. Dynamic Pile Test                       . . . . . . . . . . . . . . . . . . . . . . . . . . . Per Each

Office of Materials and Testing