# non-ms4 Detention report

PI Number: 					                Submittal Date:

Project Name: 					    Agency/Company:

City/County: 					                Let Date:

District: 				   		    Contact Phone: 			                       	 _____

Net Length of Project: ______________________miles

Existing Typical Section: _________________________

Proposed Typical Section: _______________________

Select one:

☐    I, _____________________ recommend that detention is not warranted for this project.

☐    I, ____________________ recommend that detention may be both warranted and suitable

for _________ outfall(s) on this project.

Provide Professional Engineering  Seal, Signature, Date,

Certificate of Authorization (COA) Name, COA Authorization Number, and COA Expiration Date

Note: Not required if report is prepared by GDOT

# detention guidance

Purpose of the Non-MS4 Detention Report

The purpose of the Non-MS4 Detention Report is for the designer to provide information supporting its recommendation regarding detention. The information provided must be sufficient to allow GDOT to make an informed decision whether detention should be implemented.

The goal of this decision-making process is both to protect downstream properties as much as practicable and to ensure that each proposed permanent BMP is definitively warranted. The ultimate decision whether detention is warranted and suitable shall be made by GDOT. If detention is recommended but the designer finds no BMP to be suitable, contact the Water Resources Group of the Office of Design Policy and Support.

For additional information about the manner in which this Non-MS4 Detention Report is to be completed, see the current edition of the GDOT Drainage Design for Highways Manual, Section 10.2.3 Detention Analysis & Downstream Hydrologic Assessment.

***

Attachment A

GDOT Post-Construction BMP Summary

| Outfall Number or Name | Planning Considerations |  | Location and Identification |  |  | Maintenance Responsibility |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposed BMP (Dry Detention or Wet Detention) | Is the BMP Suitable(Yes/No) | Station (Begin - End) | Offset (Left/ Right) | Plan Sheet |  |

Attachment A-1

Table 1. Pre- versus Post-Development Drainage Area Summary

| Outfall Number or Name | Pre-Development |  |  | Post-Development |  |  | Peak Flow Rate |  |  |  |  |  |  |  |  | Runoff Volumes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Drainage Area (ac) | Curve Number | Tc (min) | Drainage Area (ac) | Curve Number | Tc (min) | 10-Year |  |  | 25-Year |  |  | 100-Year (if BMP is proposed) |  |  | 25-Year |  |
|  |  |  |  |  |  |  | Pre (cfs) | Post Without Detention (cfs) | Post With Detention (cfs) | Pre (cfs) | Post Without Detention (cfs) | Post With Detention (cfs) | Pre (cfs) | Post Without Detention (cfs) | Post With Detention (cfs) | Pre (ft3) | Post (ft3) |

Attachment A-1

Table 2. Pre- versus Post-Development Downstream Analysis Summary

| Downstream Analysis Study Point | Pre-Development |  |  | Post-Development |  |  | 25-Year Peak Flow Rate |  |  |  |  |  |  | 100-Year Peak Flow Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Drainage Area (ac) | Curve Number | Tc (min) | Drainage Area (ac) | Curve Number | Tc (min) | Pre (cfs) | Post Without Detention |  |  | Post With Detention* |  |  | Pre (cfs) | Post Without Detention |  |  | Post With Detention* |  |  |
|  |  |  |  |  |  |  |  | Peak Flow (cfs) | Change (cfs) | Change (%) | Peak Flow (cfs) | Change (cfs) | Change (%) |  | Peak Flow (cfs) | Change (cfs) | Change (%) | Peak Flow (cfs) | Change (cfs) | Change (%) |

* If Post-Construction BMP is proposed, fill out the columns. Otherwise, put “N/A”.

ATTACHMENT B

Detention BMP Documentation

Georgia Department of Transportation

Detention BMP Documentation for

<Project Name>

PI No. XXXXXXXX

XXXX County

Date

Prepared By:

Agency/Company

Address

Phone number

NOTE:

Text has been included in red that gives the user instructions on how to complete the corresponding section.  This text should be removed and replaced with the information relevant to the project. The black text is example verbiage, and the blue text within the example verbiage provides specific project information to be updated.

In general, GDOT is looking for concise summaries for each drainage area evaluation. Discussion of which BMPs may be warranted, discussion of which BMP may be suitable, and (if wet detention is selected instead of dry detention) discussion of why dry detention is not recommended to be suitable. Please refer to the remainder of this template for further guidance.

Project Description

Provide the most current project description available for the project. It should include a general overview of the project and unique site conditions.

GDOT proposes to [widen I-85 to add one additional lane in each direction from just north of Sandy Cross Rd to Lavonia St.]

This project has [1] outfall(s) for which detention BMPs are recommended to be warranted and suitable. Refer to Appendix A for drainage area delineations.

Maintenance Discussion

The Design Phase Leader shall discuss the maintenance accessibility and responsibility with GDOT Maintenance/District Maintenance for a proposed BMP on a state route. If a BMP is proposed not on a state route, contact the local government to determine if they see the BMP as warranted and to see if they will accept maintenance responsibility. If the local government sees a BMP on a local route as warranted but they do not accept maintenance responsibility for the BMP, discuss with ODPS and District Maintenance before including the BMP in any milestone plan set submittal and before purchasing of Right of Way for the BMP. This section is used to document these discussions. Ensure Attachment A accurately represents the outcome of the discussions.

Area Evaluation

This section is used to discuss the evaluation process and conclusions for each drainage area where detention is recommended to be warranted and suitable. One example drainage area is shown:

Drainage Area 1 (Note: change drainage area name/number to correspond with Attachment A)

Include a brief description of the drainage area location and where it discharges. Include notes from site visit.

Drainage Area 1 is located along [road name] between station [0+00] and station [2+00]. This drainage area discharges [a short distance across a grass field within a ditch then directly into an existing 15-inch pipe under a parking lot of a business].

Physical Parameters of Drainage Area 1

Provide the pre- and post-development impervious and other land use of the drainage area used in the calculation of the flows, using the NRCS TR-55 Method. Include the hydrologic soil group and the composite curve number (CN) in the summary tables.

| Drainage Area 1 (Pre) | Area (ac) | CN |
| --- | --- | --- |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 1.25 | 61 |
| Open space - Good condition (grass cover > 75%) (Soil Group C) | 0.13 | 74 |
| Impervious | 0.35 | 98 |
| Woods - Good condition (Soil Group B) | 0.42 | 55 |
| Total | 2.15 | 67 |

| Drainage Area 1 (Post) | Area (ac) | CN |
| --- | --- | --- |
| Open space – Good condition (grass cover > 75%) (Soil Group C) | 0.13 | 74 |
| Impervious | 1.25 | 98 |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 1.25 | 61 |
| Total | 2.63 | 79 |

|  | 10-Year (cfs) | 25-Year (cfs) | 100-Year (cfs) |
| --- | --- | --- | --- |
| Pre-Development | 2.20 | 6.21 | 9.74 |
| Post-Development (unattenuated) | 9.45 | 15.40 | 21.85 |
| Change (Post - Pre) | 7.25 | 9.20 | 12.11 |
| Percent Change | 329.55% | 148.15% | 124.33% |

Warranting Criteria

State the warranting criteria which this outfall qualifies for in order to propose detention. Provide a brief (less than 1 page) narrative describing how the decision to recommend detention was achieved. Provide supporting documentation in Appendix D.

Detention is recommended for drainage area 1 based on warranting criteria #3 “Post-developed flows without detention would flood downstream property”.

Discharge from the outfall goes a short distance across a grass field within a ditch then directly into an existing 15-inch pipe under a parking lot of a business. The parking lot slopes down to the business so any overtopping of the parking lot could cause flooding of the business.

Survey data for the 15-inch pipe and the top of the parking lot has been collected. Flows have been modeled in HY-8. In the pre-developed condition, the peak flows for the 10-year and the 25-year storm safely pass through the 15-inch pipe without overtopping the parking lot. However, in the post-developed unattenuated condition the 10-year and the 25-year storm would overtop the parking lot, posing a flooding concern to the business. See Appendix D for HY-8 modeling documentation.

The right-of-way team has met with the owners of the property and discussed increasing the size of the 15-inch pipe or installing an additional pipe under the parking lot. However, the property owner stated that this is not a possibility. See Appendix D for the right-of-way team’s notes on these meetings.

As demonstrated by the topography of the location maps, the discharge cannot be bypassed around the parking lot while maintaining gravity flow. Therefore, the design team recommends installation of a post-construction BMP in the grass field between the state route and the parking lot. The pond model in Appendix G in conjunction with the HY-8 model in Appendix D demonstrate that the recommended BMP would reduce peak flows so that the 10-year storm and the 25-year storm both safely pass through the 15-inch pipe without overtopping the parking lot. Attenuated post-developed peak flow for the 100-year storm is less than the 100-year pre-developed peak flow.

Downstream Analysis

Discuss the downstream analysis of the applicable drainage area.

A downstream analysis was performed for Drainage Area 1. The downstream study point establishes a drainage area that is approximately 10 times as large as the on-site drainage area. See Appendix H for a map showing the drainage areas and downstream study point.

Discuss methodology for completing the downstream analysis. Provide drainage area characteristics used to perform the analysis.

The USGS StreamStats website was used to determine the downstream analysis drainage area along with knowledge of existing crossdrains present in the basin. The following table shows the physical parameters of the downstream analysis drainage area, not including the on-site drainage area. As the on-site drainage area is not included, the physical parameters will be the same for pre- and post-developed conditions.

| Drainage Area 1 Downstream Analysis (Minus On-site) | Area (ac) | CN |
| --- | --- | --- |
| Commercial and business (Soil Group B) | 17.22 | 92 |
| Woods - grass combination - Good condition (Soil Group B) | 3.96 | 58 |
| Total | 21.18 | 86 |

The longest flow path and the average area slope were obtained from the USGS topographic map and used to determine the time of concentration using the lag method.

| Drainage Area 1 Downstream Analysis |  |  |
| --- | --- | --- |
| L | 1486.3 | Flow Length (ft) |
| CN | 86 |  |
| Y | 9.8 | Watershed Slope (%) |
| S | 1.63 | Maximum Retention (in) |
| Tc | 0.19 | Hours |
| Tc | 11.4 | Minutes |

Model the timing of the hydrographs with and without the detention BMP. Due to peak flow timing and runoff volume effects, some structural practices fail to reduce discharge peaks to pre-development levels downstream from the development site and in certain cases may actually exacerbate flooding problems. A downstream peak flow analysis shall be provided to the point in the watershed downstream of the site or the stormwater management system where the area of the site comprises approximately 9-11% of the total drainage area or where discharges from the project enter a stream or waterbody that is large enough for the site discharges to become negligible. This is to help ensure that there are minimal downstream impacts from the developed site. The downstream analysis may result in the need to resize BMPs, or may allow the waiving of some peak flow controls altogether.

The channel routing function in Hydraflow Hydrographs was then used to model the timing of the hydrographs. See Appendix G for copies of the hydrographs.

|  | Pre-Development | Without BMP |  |  | With BMP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Post-Development | Change |  | Post-Development | Change |  |
|  |  |  | (cfs) | (%) |  | (cfs) | (%) |
| 25-Year Peak Flow (cfs) | 118.78 | 123.76 | 4.98 | 4.19% | 121.56 | 2.78 | 2.34% |
| 100-Year Peak Flow (cfs) | 154.58 | 159.74 | 5.16 | 3.34% | 157.70 | 3.12 | 2.02% |

Best Management Practice (BMP) Assessment Selection

First assess if a dry detention basin is appropriate BMP for the drainage area.

Dry Detention Basin – The dry detention basin is an appropriate BMP for this drainage area.

Suitability

For the BMP that was deemed appropriate for the drainage area based on meeting design constraints and requirements, assess the suitability per GDOT’s seven criteria. Provide BMP sizing calculations, displays, etc. in the appendices of the report to justify the claim(s).

Dry Detention Basin – The dry detention basin is suitable for this drainage area.

Check each box to certify that the proposed BMP is suitable.

☐ The project would not require an existing roadway alignment change or a safety concern solely to allow for BMPs.

☐ The installation of post-construction BMPs would not require the re-alignment and/or piping of a stream.

☐ Implementation of BMPs would not cause loss of habitat for endangered or threatened species.

☐ Implementation of the BMP would not cause significant damage to cultural or community resource such as an historical site, archeological site, cemetery, park, wildlife refuge, nature trail, or school facilities.

☐ Implementation of BMPs would not result in violation of state or federal law or regulation.

☐ There are no foreseen site limitations including: shallow bedrock, contaminated soils, high groundwater, utilities, or underground facilities if avoidance or relocation is not suitable.

☐ Site does allow for gravity flow to the appropriate BMP.

Drainage Area 1 was found to be suitable for a dry detention basin.

Appendix A –Drainage Area Maps

Include maps showing the pre- and post-development drainage areas.

Appendix B – NOAA Precipitation Table

Include a print out of the NOAA precipitation table for the midpoint of the project (https://hdsc.nws.noaa.gov/hdsc/pfds/pfds_map_cont.html?bkmrk=ga)

Appendix C – Soils Map

Include soil map(s) showing the hydrologic soil group in each drainage area.

Appendix D – Warranting Criteria

Include documentation to support the recommendation that detention is warranted. Include relevant supporting survey (elevations of buildings, driveways, etc.), relevant hydraulic modeling (of channels or pipes downstream), relevant meeting minutes, and any other supporting documentation.

Appendix E – Hydrologic Model Output or PondPack Model

Provide the hydrologic model output or the PondPack model for each drainage area. Include the output for the 10-year, 25-year, 100-year 24-hour storms, and any other pertinent storm event. Although TR-55, Hydraflow, or HydroCAD can be used, GDOT only has access to PondPack so either hydrographs or the PondPack model need to be provided. For authorization to use other software in preparation of the report, contact ODPS.

Appendix F – Downstream Analysis Documentation

Provide the backup documentation supporting the downstream analysis for each drainage area. Provide:

- Drainage area maps and aerial photography defining existing and proposed drainage areas indicating topography and flow paths

- Field notes, photographs, and any correspondence with local residents or other contacts

- Flood maps (if available)

- Hydrographs from TR-55, Hydraflow, or HydroCAD and model inputs/outputs (or the PondPack model).

Appendix G – Cost Calculations

Provide a cost estimate for each recommended BMP (per each) and cost estimate for ROW acquisition for each recommended BMP. Using the tables below, fill out a separate table for each recommended BMP as ROW costs will be different for different BMPs. The designer may use AASTOWare Estimation, or item mean summary data to estimate the per each cost of BMPs. Assume $5,000 for landscaping on wet detention basins. Please assume the maintenance costs pre-populated in the tables unless there are extenuating circumstances (describe the circumstances if extenuating circumstances exist).

Add a row in the table for the cost of utilities relocations if needed to be relocated specifically for the basin. Contact District Utilities Coordinator to get the cost of utilities relocations. If a new wall is needed to be constructed specifically for the basin, add a row in the table for it as well.

|  | Dry Detention Basin # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Dry Detention | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $2,000 | $40,000 |
|  |  |  | Total |  |

|  | Wet Detention Basin # |  |  |  |
| --- | --- | --- | --- | --- |
|  | Unit | Quantity | Unit Cost | Total Cost |
| Wet Detention | Each | 1 |  |  |
| ROW | Acres |  |  |  |
| Maintenance for 20 years | Per Year | 20 | $3,000 | $60,000 |
| Landscaping | Lump Sum | 1 | $5,000 | $5,000 |
|  |  |  | Total |  |

Appendix H – Construction Plan Sheets

Provide copies of draft construction plan sheets for each recommended post-construction BMP. ODPS acknowledges that the plan sheets submitted in this appendix are not finalized. As previously mentioned, the detention report shall be accepted prior to including post-construction detention in any milestone plan set. The plans within this appendix will be reviewed against the pond model, the requirements within GDOT’s Drainage Design for Highways Manual, and the requirements within GDOT’s special construction details.

Plan presentation comments will be made separately on milestone submittals which have post-construction BMPs. If the designer chooses to provide an interim plan submittal between milestones, ODPS can review the interim plans after the detention report is accepted by ODPS.