# MS4 post-construction stormwater report

PI Number:

Project Name:

City/County:

District:

Submittal Date:

Agency/Company:

Let Date:

Contact Phone:

Contact Email:

Milestone Submittal: ☐PCSR (First draft)      ☐PCSR (Resubmittal)       ☐ PCSR Addendum

Project Delivery: ☐ Design-Bid-Build    ☐ Design-Build Costing Phase   ☐ Design-Build Final Phase

Applicable MS4 Permit: ☐  2012     ☐  2017    ☐  2022, No EPD Submittal    ☐  2022, EPD Submittal

General Project Information:

Is there a Project Level Exclusion that applies to this project:    ☐  Yes   ☐  No

If yes, please indicate which of the following exclusions apply:

☐  1. Roadway not owned or operated by GDOT.

2. Project location is not in an MS4 area. (Submittal not required)

3. Maintenance or safety project. (Submittal not required)

☐  4. Project with environmental documents approved or R/W plans submitted on or before June 30, 2012.

☐  5. Road project disturbing < 1 acre.

☐  6. Site development/redevelopment project that creates, adds, or replaces < 5,000 ft2 of impervious area.

☐  7. Project in MS4 area added to the 2017 MS4 permit with concept approval before January 3, 2018.

☐  8. Project in Combined Sewer Overflow area.

Disturbed Area of Site: _________________acres    Existing Typical: Urban* ☐   Rural ☐  No. Lanes______

Impervious Area Added: _______________acres    Proposed Typical: Urban* ☐ Rural ☐  No. Lanes_____

Net Length of Project: __________________miles     *In this instance, “urban” means curb and/or gutter.

Discharge Information:

Yes /  No

☐     ☐    Does the project discharge to a trout stream?

☐     ☐    Does the project discharge to a stream impaired for Bio F or Bio M?

Submittal Requirements:

Yes /  No

☐     ☐    GDOT LID / GI Checklist (Attachment A)

☐     ☐    GDOT Post-Construction BMP Summary (Attachment B)

☐     ☐    Pre- vs Post-Development Drainage Area Summary (Attachment B-1)

☐     ☐    Post-Construction Stormwater BMP Documentation (Attachment C)

PE Seal, Signature, & Date

Note: Not required if report is prepared by GDOT. Not required if Design-Build Costing Phase.

☐     ☐    Milestone Plan Submittal Checklist (Attachment D)

# MS4 post-construction stormwater decision tree

Complete the cover page of the MS4 Post-Construction Stormwater Report and submit required supporting documentation. Assess if Non-MS4 Detention Report is needed.

Is the project  maintenance or safety? (not including an RCUT nor a roundabout)

Is the project located in an MS4 area?

Is there a Project Level Exclusion that applies to this project?

No

Complete the “Stormwater BMP Infiltration Worksheet(s)” column in Attachment B. See Appendix J of the Drainage Manual for worksheet(s). Place the worksheet(s) in PCSR Appendix K.

Indicate on Attachment B if there is an Outfall Level Exclusion. See Drainage Manual section 10.3.2 for reference.

Complete the “Drainage Area Characteristics” columns on Attachment B – Post-Construction BMP Summary Table.

Identify the appropriate BMP(s) for each outfall drainage area that does not have an exclusion. See Drainage Manual section 10.5.4 for reference.

No MS4 Post-Construction Stormwater Report documentation is required. Assess if Non-MS4 Detention Report is needed.

Notify the MS4 Review Team and Stormreports Inbox of the project so it can be added their tracking spreadsheet.

Yes

No

Yes

No

Yes

Put “No” in the “Stormwater BMP Infiltration Worksheet(s)” column in Attachment B.

Complete the “Feasible BMP(s)”, “Location and Identification” and “Maintenance Responsibility” columns in Attachment B.

Complete the “Infeasible BMP(s)” and “Applicable Infeasibility” columns in Attachment B.

Yes

No

Complete Attachment C – Post-Construction Stormwater BMP Documentation and Attachment D – Milestone Plan Submittal Checklist.

No

Are any BMP(s) considered Infeasible?

Yes

Is an infiltration BMP being considered for a drainage area?

Calculate the pre- and post-development flows and complete the “Applicable MS4 Requirements” columns on Attachment B-1.

Delineate pre- and post-development drainage basins.

Complete Attachment A – LID/GI Checklist.

Complete the cover page of the MS4 Post-Construction Stormwater Report & continue the evaluation.

# MS4 post-construction stormwater guidance

Training materials are available on the Design Manuals and Guides website https://www.dot.ga.gov/GDOT/pages/DesignManualsGuides.aspx and classes are available on the GDOT Learning Management System (LMS) website http://www.dot.ga.gov/PS/Training . Please refer to this material prior to completing a MS4 Post-Construction Stormwater Report (PCSR).

The MS4 PCSR should be a standalone document.  All relevant information should be included in the MS4 PCSR for review and reproduction of results/conclusions.  The MS4 PCSR, however, should only include necessary information (i.e., 1-, 25-, and 100-year input/output data).

The MS4 PCSR should initially be submitted to GDOT in PDF format in lieu of hard copy for the GDOT Office of Design Policy & Support’s (ODPS) review. It is not necessary for a Professional Engineer (PE) to sign and seal the Report until it has been accepted by GDOT. A memo will be issued when the MS4 PCSR has been accepted. After the acceptability memo is received, submit to ODPS a PDF copy of the Report with the cover signed, sealed, and dated by a PE (PE seal not required if prepared by GDOT designers).

MS4 PCSR Addendum Process

As shown in the MS4 Plan Development Process Flowchart, an addendum may be required to be submitted if there are project changes after the MS4 PCSR has been accepted and considered final. An addendum is required when:

- An outfall not previously considered has been identified,

- An outfall previously considered infeasible becomes feasible,

- An outfall previously considered feasible is now infeasible,

- The type of post-construction BMP at an outfall has changed, and/or

- The performance of a post-construction BMP to be built is significantly different than that modeled in the previously accepted PCSR e.g. the drainage area to the BMP is at least 20% different or the outlet structure design is significantly different.

If required, the addendum only needs to address the drainage basins that have changed. Submit the following as a draft in PDF format to ODPS for review:

- Cover letter outlining the changes

- Revised cover page

- Revised Attachment B and Attachment B-1

- Revised sections and associated backup documentation in Attachment C

- Revised appendices, as relevant to the changes to the outfall or BMP

- Current construction plans

- Stormwater BMP Infiltration Worksheet(s) (if applicable)

When ODPS accepts the addendum, submit to ODPS a PDF copy of the addendum with the cover signed, sealed, and dated by a PE (PE seal not required if prepared by GDOT designers).

SUMMARY OF LID/GI PRACTICES

Included as part of post-construction stormwater design, Low Impact Development and Green Infrastructure practices are required to be considered by the designer during the stormwater planning process for each GDOT project within an MS4 area.

Practices of LID include the following:

- Avoiding environmentally sensitive areas

- Reducing the project footprint

- Minimizing site impacts

- Adjusting the design with the natural terrain

- Incorporating water quality early in planning

- Recycled materials

- Green infrastructure practices

Practices of GI include the following:

- Infiltration

- Evapotranspiration

- Porous Pavements

- Reuse

All post-construction stormwater BMPs are considered LID/GI with the exception of the dry detention basin.

As part of GDOT’s policy on how to consider LID and GI practices, the designer must always attempt to utilize LID/GI practices when it is both feasible and within an MS4 area.  During the planning process, it is important for the designer to consider the site constraints and limitations for the LID/GI practices being considered.  The designer is required to complete the LID/GI Checklist to document which specific elements have been used and the reasoning for each case.  Refer to Attachment A for this checklist.  For additional information, see Chapter 10 of the GDOT Drainage Design for Highways Manual (Drainage Manual).

GDOT POST-CONSTRUCTION SUMMARY

Attachment B is the GDOT Post-Construction BMP Summary.  This form must be completed by the designer and submitted with all stormwater reports without a Project Level Exclusion at milestone reviews.  The purpose of the documentation is to assist GDOT in inventorying new post-construction stormwater BMPs and also assist with the plan review process.

STORMWATER RUNOFF QUALITY/REDUCTION

GDOT’s 2017 MS4 Permit added the requirement to retain up to the first 1.0 inch of rainfall on the site to the maximum extent practicable. If the 1.0 inch cannot be retained onsite, the remaining runoff from a 1.2 inch rainfall event must be treated to remove at least 80% of the calculated average annual post-development Total Suspended Solids (TSS) load. The requirement to retain up to the first 1.0 inch of rainfall on the site to the maximum extent practicable applies to projects that receive concept approval January 3, 2018 or later. For projects that receive concept approval prior to January 3, 2018:

- If the project is located in an MS4 area that was covered under 2012-2017 MS4 Permit, the project must meet the post-construction stormwater requirement in the 2012 MS4 Permit to treat the runoff from the 1.2 inch rainfall event to remove at least 80% TSS.

- If the project is located in an MS4 area that was added to the 2017 MS4 Permit, the project is exempt from MS4 post-construction stormwater requirements.

For additional information, see Chapter 10 of the GDOT Drainage Design for Highways Manual (Drainage Manual).

FEASIBILITY & EXCLUSION OF POST-CONSTRUCTION BMPs

There are exclusions (or certain instances) where GDOT does not require post-construction stormwater BMPs.  That list of exclusions includes both “Project Level” and “Outfall Level” exclusions.  The Project Level Exclusions (PLE) will eliminate the need for the design or construction of post-construction BMPs for the entire project area, whereas the Outfall Level Exclusions (OLE) can only be claimed on a per outfall basis.  The MS4 outfall is the most downstream point on an MS4 drainage area where it discharges to waters of the State. It does not include cross-drain structures or culverts installed under a road that function only to maintain the natural flow of surface waters and drainage. However, a structure that collects or diverts drainage that has contacted road surfaces for discharge into waters of the State is considered an outfall under this Permit. In addition, wherever a water feature leaves the right-of-way (ROW) prior to entering waters of the State, the point at which the water feature leaves the ROW is considered the MS4 outfall for the purpose of this report.

Project Level Exclusions requiring the submittal of the MS4 PCSR cover and supporting documentation include the following:

- Roadways that are not owned or operated (maintained) by the Department may not require post-construction BMPs.  Coordination with the local government is necessary for determination.

- Projects that have their environmental documents approved or R/W plans submitted for approval on or before June 30, 2012.

- Road projects that disturb less than 1 acre.

- Site development/redevelopment projects that create, add, or replace less than 5,000 square feet of impervious area.

- Projects in MS4 areas added to the 2017 MS4 permit with concept approval before January 3, 2018.

- Projects that discharge to a Combined Sewer Overflow area.

Outfall Level Exclusions include the following:

- Cases where the project would require an existing roadway alignment change solely to allow for BMPs. This exclusion applies only to existing roadway alignment changes that would create a safety concern. A written explanation of the safety concern(s) must be included with the post-construction stormwater report for all uses of this exclusion.

- Instances where the installation of post-construction BMPs would require the re-alignment and/or piping of a stream.

- When a project would impact existing vegetated stream buffers or wetlands solely for the purposes of installing BMPs.  See state requirements for additional information on stream buffers.

- Where stormwater discharges from the project site are designed to exit the right-of-way or enter a state water as sheet flow (non-point source discharges).  Sheet flow should be designed in a manner to ensure that the flow will not cause instability, erosion, or flooding.  The designer should determine if this is possible by visiting the site prior to design, and providing a written explanation with supporting evidence for this drainage area.

- As stated in section 4.2.5.1(a) of the GDOT MS4 permit, “Stormwater runoff that must be treated does not apply to flows that originate outside of GDOT’s right-of-way or diverted flows from undisturbed areas.”  If feasible, direct all offsite stormwater around the project site to the cross drain or stream such that it does not combine with stormwater from the project’s impervious surfaces or conveyance systems.  This redirection allows the BMPs to only treat or detain the stormwater that originates from GDOT’s site, and stormwater that originates off-site to pass through the right of way unimpeded.

- As stated in section 4.2.5.1(a) of the GDOT MS4 permit, for outfalls along linear roadway projects whereby the net impervious surface area within that outfall’s drainage area has been reduced or remains the same as pre-developed conditions, post-construction stormwater requirements will not apply. Special consideration from the Department may be given to those projects with a minimal increase in impervious area. In such cases, the designer will be required to provide supporting calculations showing that the increase in stormwater runoff and/or volume required to be treated for water quality is negligible with respect to the drainage area in question. As a general rule, increases over one tenth of an acre in impervious surface per basin are not considered negligible.

Exclusions should be noted on Attachment B within the “Applicable MS4 Requirements” section.  For simplicity, record the number from the list above corresponding to the exclusion being claimed.

In some situations, site constraints and other factors may make installing BMPs to meet each unified sizing criteria infeasible. The following criteria are used to define the situations where implementing post-construction BMPs is considered infeasible and should be applied to each outfall area individually:

- The BMP costs equal or exceed 10% of the total project costs within the drainage area to the outfall. If the BMP discharges within one linear mile upstream of and within the same watershed as a designated trout stream then the threshold for BMP cost infeasibility is 30% of the total project costs within the drainage area to the outfall. Project costs should include:

  - Right-of-way acquisition

  - Roadway construction (not including Intelligent Transportation Systems (ITS) or toll related expenses)

  - Utility relocation

  - Mitigation costs

BMP costs should include:

  - Additional right-of-way requirements

  - BMP construction

  - All other elements as needed specifically for the BMP such as maintenance access, utility relocation, landscaping, etc., as applicable

- Implementation of BMPs will cause 90 days or greater of delays to the project. This criteria does not apply to projects that discharge within one linear mile upstream of and within the same watershed as a designated trout stream.

- Implementation of BMPs will cause loss of habitat for endangered or threatened species.

- Implementation of BMPs will cause significant damage to cultural or community resource such as an historical site, archeological site, cemetery, park, wildlife refuge, nature trail, or school facilities.

- Implementation of BMPs would result in the displacement of a residence or business.

- Implementation of BMPs would result in violation of state or federal law or regulation.

- Site limitations including: shallow bedrock, contaminated soils, high groundwater, utilities, or underground facilities if avoidance or relocation is infeasible (for example, if cost of the relocation equals or exceeds the cost of the BMP).

- Soil infiltration capacity is limited, where the soil hydraulic conductivity (K) is less than 10-4 cm/second.

- Site is too small to infiltrate a significant volume.

- Site does not allow for gravity flow to the appropriate BMP.

GDOT requires that downstream properties be evaluated for damages from increased flows and that downstream conveyances be evaluated for sufficient capacity for increased flows. The need for detention facilities should be determined on a case-by-case basis and their use may not be required on certain projects. However, it is the designer’s responsibility to provide all necessary supporting documentation for a detention analysis, per outfall, as to whether detention is needed or not. An analysis for detention may include, but is not limited to, factors such as:

- Increase in peak flow rates

- Downstream conveyance capacity

- Environmental impacts

- Downstream detention facilities

If a downstream analysis indicate detention is required to mitigate adverse downstream impacts, detention must be provided, regardless of MS4 exclusions or infeasibilities. The protection of property downstream of a GDOT project or facility is a vital concern that must be addressed.

Current GDOT policy provides for the following BMPs for post-construction stormwater management:

|  |  | Treatment Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BMP |  | RRv | WQV | TSS Removal | CPV | QP25 | Qf |
| Filter Strip |  | 25% | Yes | 60% | No | No | No |
| Grass Channel | A & B HSG | 25% | Yes | 50% | No | No | No |
|  | C & D HSG | 10% |  |  |  |  |  |
| Enhanced Dry Swale |  | 50% | Yes | 80% | In Some Situations | No | No |
| Enhanced Wet Swale |  | 0% | Yes | 80% | In Some Situations | No | No |
| Infiltration Trench |  | 100% | No | 100% | In Some Situations | No | No |
| Sand Filter |  | 0% | Yes | 80% | In Some Situations | No | No |
| Dry Detention Basin |  | 0% | Yes | 60% | Yes | Yes | Yes |
| Wet Detention Pond |  | 0% | Yes | 80% | Yes | Yes | Yes |
| Stormwater Wetland – Level 1 |  | 0% | Yes | 80% | Yes | Yes | Yes |
| Stormwater Wetland – Level 2 |  | 0% | Yes | 85% | In Some Situations | No | No |
| Bioslope |  | 25% | Yes | 85% | No | No | No |
| Bioretention Basin | Open underdrain | 50% | Yes | 85% | In Some Situations | No | No |
|  | Upturned underdrain | 75% |  | 100% |  |  |  |
|  | Capped underdrain | 100% |  |  |  |  |  |
| Open Graded Friction Course (OGFC) |  | 0% | Yes | 80% | No | No | No |
| Regenerative Stormwater Conveyance |  | 0% | Yes | 80% | No | No | No |

TSS = Total Suspended Solids

RRv = Runoff Reduction Volume

WQv = Water Quality Volume

CPv = Channel Protection Volume

Qp25 = Overbank Flood Protection

Qf = Extreme Flood Protection

HSG = Hydrologic Soil Group

As shown in the table above, certain BMPs do not provide all treatment required and would have to be used in a “treatment train.” If used in a treatment train, the TSS removal for the treatment train would be calculated by using 100% of the TSS removal of the first BMP in the treatment train and remaining TSS times the TSS removal rate of the second BMP in the treatment train. For example, if a filter strip and a grass channel were used together the total TSS removal based on the table above would be 60% (for the filter strip) + (40% remaining TSS)* 50% (for the grass channel) which would result in a total TSS removal of 80%.

In accordance with the GDOT MS4 permit and guidelines, appropriate BMPs that can meet the runoff reduction criteria must be evaluated first. BMPs with the least amount of impact should be evaluated next. The least amount of impact is defined as the lowest cost BMP with the lowest long term maintenance cost that will provide the required treatment for the drainage area. Note that the long term maintenance cost of a BMP is used when determining the order of appropriate BMP evaluations and should not be used to justify why a BMP is not appropriate for a drainage basin or as an infeasibility criteria. If determined inappropriate or infeasible, the next BMP with the least amount of impact will be considered until an appropriate BMP is selected or all of the BMPs are eliminated.

Post-Construction Stormwater BMP Documentation

Attachment C of the MS4 Post-Construction Stormwater Report is where the designer will document in detail each basin evaluation. This document will detail the outfall level exclusions and infeasibilities, as well as the feasible BMP designs. All supporting information (cost estimates, figures, calculations, and other backup data) must be included with the Post-Construction Stormwater BMP Documentation.

The template is meant to serve as a guide and will not encompass all projects.  It is intended to be flexible and if changes to the report are needed for a specific project, the engineer of record should exercise their judgement when making changes from the template format.  To use the BMP Documentation template, text has been included in red that gives the user instructions on how to complete the corresponding section.  This text should be removed and replaced with the information relevant to the project. The black text is example verbiage, and the blue text within the example verbiage provides specific project information that will need to be updated.

MILESTONE SUBMITTALS

GDOT has developed the Milestone Plan Submittal Checklist to be used by designers as a guide to the stormwater documentation that is required for the major milestone submittals.  Refer to Attachment D.  This checklist is required by GDOT, and is expected that all information be included with the plan submittal at the time of review.

Attachment A

GDOT Low Impact Development (LID) / Green Infrastructure (GI) Checklist

Design Considerations

☐  The following site considerations were considered, where applicable, and incorporated into an LID/GI approach: safety, ease of maintenance, available right-of-way, soils, terrain slope, pollutants of concern, existing utilities and other infrastructure details

☐  Where applicable, the following site-specific environmental components have been clearly identified on the project site: wetlands, impaired waters, environmentally sensitive areas, applicable buffers

Design Documentation

List any site-specific limitations or constraints that will have an effect on the utilization of feasible post-construction stormwater LID and/or GI practices.

☐  The following LID/GI practices were used.  For those that were not used, explain why it was infeasible for this project.

Yes    No

☐    ☐    Avoidance (Planning around environmentally sensitive areas):

☐    ☐    Minimization:

☐    ☐    Footprint reduction:

☐    ☐    Incorporating WQ early in planning process: ____________________________________________________

☐    ☐    Rural road section in place of urban: ___________________________________________________________

☐    ☐    Landscaping areas outside of clear-zone w/ trees:

☐    ☐    Adjusting the design to natural terrain: _________________________________________________________

☐    ☐    Porous Pavements (OGFC):

☐    ☐    Post-construction BMPs that allow for: infiltration, evapotranspiration, and stormwater reuse

☐    ☐    Using recycled materials such as asphalt and concrete:

☐  The LID/GI practices shown on the plans address all GDOT and MS4 permit requirements

☐  A cost estimate has been provided to GDOT

Inspection and Maintenance Responsibility (select all that apply)

☐  Dedicated to City or County (indicate which) of:

☐  Private Entity Responsibility: name responsible entity here:

☐  GDOT Responsibility

Attachment B

GDOT Post-Construction BMP Summary

| Drainage Area Characteristics |  |  |  |  |  | Applicable MS4 Requirements |  |  |  |  |  |  | Planning Considerations |  |  |  |  |  | Location and Identification |  |  |  | Maintenance Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outfall | Receiving Water | Trout Stream (Yes or No) | Impaired (Yes or No) | Impairment | Approved TMDL (Yes or No) |  | OLE (Yes/ No) (If yes, see Note 1) | RRv ( or X) | WQv ( or X) | CPv ( or X) | Qp25 ( or X) | Qf ( or X) |  | Infeasible BMP(s) | Applicable Infeasibility (see Note 2) | BMPs to be Built | Stormwater BMP Infiltration Worksheet(s) (Yes/No) (See Note 3) | BMP(s) required for commitments to another agency? (Yes/No) |  | Station (Begin - End) | Offset (Left/ Right) | Plan Sheet |  |

Note 1: If an Outfall Level Exclusion is claimed, include the exclusion number (as listed in the Post-Construction Stormwater Report Guidance section) and provide supporting evidence in Attachment C.

Note 2: If a BMP is identified as infeasible, include the infeasibility number (as listed in the Post-Construction Stormwater Report Guidance section) and provide supporting evidence in Attachment C.

Note 3: See Appendix J of the GDOT Drainage Design for Highways Manual for guidance on infiltration testing and the Stormwater BMP Infiltration Worksheet(s).

Attachment B-1

Pre- versus Post-Development Drainage Area Summary

| Outfall | Pre-Development |  |  | Post-Development |  |  | Peak Flow Rate |  |  |  |  | Runoff Volumes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Drainage Area (ac) | Curve Number | Tc (min) | Drainage Area (ac) | Curve Number | Tc (min) | 1-Year | 25-Year |  | 100-Year |  | 25-Year |  | 100-Year |  |  |
|  |  |  |  |  |  |  | Post Unattenuated (cfs) | Pre (cfs) | Post Unattenuated (cfs) | Pre (cfs) | Post Unattenuated(cfs) | Pre (ft3) | Post (ft3) | Pre (ft3) | Post (ft3) |  |

ATTACHMENT C

Post-Construction Stormwater BMP Documentation

Georgia Department of Transportation

Post-Construction Stormwater BMP Documentation for

<Project Name>

PI No. XXXXXXXX

XXXX County

Date

Prepared By:

Agency/Company

Address

Phone number

NOTE:

In general, GDOT is looking for concise summaries for each basin evaluation. Discussion of which basins have an OLE, which BMPs were feasible, which BMPs were infeasible, why the BMPs were selected, and documentation supporting the determination with calculations and/or drawings should be included in this attachment.  Please refer to the remainder of this template and the MS4 PCSR Help File for additional guidance.

Executive Summary

In general, the Executive Summary should state background information.  It should summarize the evaluation process and the results of the evaluation.

In January 2012, the Environmental Protection Division (EPD) of the Georgia Department of Natural Resources issued the Georgia Department of Transportation’s (GDOT’s) first Municipal Separate Storm Sewer System (MS4) Permit (General NPDES Permit No. GAR041000) (Permit) for discharges from its MS4 designated areas. In January 2017, EPD issued GDOT’s second MS4 Permit. In January 2022, EPD issued GDOT’s third MS4 Permit.

The Permit regulates new and existing point source discharges of stormwater from roadways owned and operated by GDOT to waters of the State of Georgia. The [SR 1 widening] (Project) must meet the requirements of the Permit, which include incorporating permanent water quality control and detention measures (best management practices [BMPs]) into the design where appropriate, where those BMPs have not been determined to be infeasible based on the exclusion and infeasibility criteria identified in Chapter 10 of the GDOT Drainage Design for Highway Manual.

To assist with the development of final design for the project and meet Permit requirements, [firm name] performed an analysis of the project in accordance with the guidance and criteria discussed above and below to identify and size feasible post-construction stormwater BMPs that must be implemented and those that may be eliminated.

This report documents the applicable guidance and criteria, analysis performed, and results and conclusions. The analysis is based on current design and cost of the improvements. Feasibility of the post-construction BMPs will need to be revisited during the final design and revised based on the revised project design or refined cost estimates.

Project Description

Provide the most current project description available for the project. It should include a general overview of the project and unique site conditions.

GDOT proposes to [widen SR 1 to add one lane in each direction from just north of [insert side road name] to [insert other side road name].

Refer to the location map in Appendix A which illustrates the approximate project limits.

This project is divided into [8] proposed drainage basins. Refer to Appendix A for drainage basin delineations.

Maintenance Discussion

The Design Phase Leader should discuss the maintenance accessibility and responsibility with GDOT Maintenance/District Maintenance for a selected BMP including the consideration and use of local municipal maintenance forces and required agreements.  This section is used to document this discussion. Ensure the Inspection and Maintenance Responsibility information in the LID/GI Checklist and Attachment B accurately represent the outcome of the discussion.

Basin Evaluations

This section is used to discuss the evaluation process and conclusions for each drainage basin. Three example basins are shown: Drainage Basin 1 requires an infeasibility assessment, Drainage Basin 2 has an outfall level exclusion, and Drainage Basin 3 requires infeasibility assessment and warrants detention.

Drainage Basin 1 (Note: change basin name/number to correspond with Attachment B)

Include a brief description of the drainage basin location and where it discharges.

Drainage Basin 1 is located along [road name] between station [0+00] and station [2+00]. This drainage basin discharges [directly into an existing drop inlet and 15-inch reinforced concrete pipe that discharges outside of the right-of-way].

Physical Parameters of Drainage Basin 1

Provide the pre- and post-development impervious and other land use of the basin used in the calculation of the flows, using the NRCS TR-55 Method. Include the hydrologic soil group and the composite curve number (CN) in the summary tables. Provide program input and output used to calculate the peak discharges in an appendix of the report.

The land use tables should be for the total area to the outfall, unless detention is proposed. If detention is proposed then the post land use table should be separated into onsite and offsite areas.

| Drainage Basin 1 (Pre) | Area (ac) | CN |
| --- | --- | --- |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 0.25 | 61 |
| Open space - Good condition (grass cover > 75%) (Soil Group C) | 0.13 | 74 |
| Impervious | 1.35 | 98 |
| Woods - Good condition (Soil Group B) | 0.42 | 55 |
| Total | 2.15 | 84 |

| Drainage Basin 1 (Post) | Area (ac) | CN |
| --- | --- | --- |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 0.02 | 61 |
| Impervious | 1.71 | 98 |
| Woods - Good condition (Soil Group B) | 0.42 | 55 |
| Total | 2.15 | 89 |

The peak flow table should be for the total area to the outfall.

|  | 1-Year (cfs) | 25-Year (cfs) | 100-Year (cfs) |
| --- | --- | --- | --- |
| Pre-Development | 4.84 | 14.69 | 17.69 |
| Post-Development | 5.99 | 15.93 | 18.89 |
| Change (Post - Pre) | 1.15 | 1.24 | 1.20 |
| Percent Change | 23.76% | 8.44% | 6.78% |

If an outfall level exclusion does not apply, this section is used to discuss the feasibility of installing a BMP in the drainage basin. First, evaluate the applicable unified sizing criteria volumes. The data in this table should be for the onsite area.

Water Quality and Channel Protection

| Total Drainage Area (ac) | 2.15 |
| --- | --- |
| Pre-Developed Impervious Area (ac) | 1.35 |
| Post-Developed Impervious Area (ac) | 1.71 |
| Pre-Developed % Impervious | 62.79 |
| Post-Developed % Impervious | 79.53 |
| Runoff Coefficient (Rv) | 0.151 |
| Target RRv (ft3) | 1,178 |
| Target WQv (ft3) | 1,411 |
| Target CPv (ft3) | 3,115 |

Supporting stormwater runoff quality/reduction (runoff reduction volume and water quality volume) and channel protection volume calculations are included in Appendix E and Appendix F, respectively.

Downstream Analysis

Discuss the downstream analysis of the applicable basin.

A downstream analysis was performed for Drainage Basin 1. The downstream study point establishes a basin that is approximately 10 times as large as the on-site basin. See Appendix H for a map showing the drainage basins and downstream study point.

Discuss methodology for completing the downstream analysis. Provide basin characteristics used to perform the analysis.

The USGS StreamStats website was used to determine the downstream analysis drainage basin along with knowledge of existing crossdrains present in the basin. The following table shows the physical parameters of the downstream analysis basin, not including the on-site basin. As the on-site basin is not included, the physical parameters will be the same for pre- and post-developed conditions.

| Drainage Basin 1 Downstream Analysis (Minus On-site) | Area (ac) | CN |
| --- | --- | --- |
| Commercial and business (Soil Group B) | 17.22 | 92 |
| Woods - grass combination - Good condition (Soil Group B) | 3.96 | 58 |
| Total | 21.18 | 86 |

The longest flow path and the average basin slope were obtained from the USGS topographic map and used to determine the time of concentration using the lag method.

| Drainage Basin 1 Downstream Analysis |  |  |
| --- | --- | --- |
| L | 1486.3 | Flow Length (ft) |
| CN | 86 | Curve Number |
| Y | 9.8 | Watershed Slope (%) |
| S | 1.63 | Maximum Retention (in) |
| Tc | 0.19 | Hours |
| Tc | 11.4 | Minutes |

Model the timing of the hydrographs with and without the detention BMP if detention is recommended. Due to peak flow timing and runoff volume effects, some structural practices fail to reduce discharge peaks to pre-development levels downstream from the development site and in certain cases may actually exacerbate flooding problems. A downstream peak flow analysis shall be provided to the point in the watershed downstream of the site or the stormwater management system where the area of the site comprises approximately 9-11% of the total drainage area or where discharges from the project enter a stream or waterbody that is large enough for the site discharges to become negligible. This is to help ensure that there are minimal downstream impacts from the developed site. The downstream analysis may result in the need to resize BMPs, or may allow the waiving of some peak flow controls altogether.

The channel routing function in Hydraflow Hydrographs was then used to model the timing of the hydrographs. See Appendix G for copies of the hydrographs.

|  | Pre-Development | Without BMP |  |  | With BMP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Post-Development | Change |  | Post-Development | Change |  |
|  |  |  | (cfs) | (%) |  | (cfs) | (%) |
| 25-Year Peak Flow (cfs) | 118.78 | N/A | N/A | N/A | 121.56 | 2.78 | 2.34% |
| 100-Year Peak Flow (cfs) | 154.58 | N/A | N/A | N/A | 157.70 | 3.12 | 2.02% |

A downstream analysis is needed for all MS4 outfalls unless the drainage area qualifies for OLE #6 based on minimal net new impervious area (less than or equal to 0.1 acre) and total post drainage area remains within 0.1 acre of the total pre drainage area (and unless the basin is included in the downstream analysis for another basin).

There is not a significant increase in flow rates at the downstream analysis study point between pre- and post-development conditions. The existing drainage system has enough capacity to handle the small increase in flow rates with no adverse impacts. Therefore, no detention is required for overbank or extreme flood protection for Drainage Basin 1.

Warranting Criteria

State the warranting criteria from Section 10.2.3 of the Drainage Manual which this outfall qualifies for in order to propose detention. Provide a brief (less than 1 page) narrative describing how the decision to recommend detention was achieved. Provide supporting documentation in Appendix H.

None of the warranting criteria were met from Section 10.2.3 of the Drainage Manual, so detention is not recommended for downstream risk mitigation purposes. However, feasibility for the MS4 unified sizing criteria will still be assessed.

Best Management Practice (BMP) Selection

Assess each of the GDOT BMPs for appropriateness for the basin. Refer to Table 10.5-1 BMP Screening Criteria in the Drainage Manual for additional guidance. Selection criteria for choosing an appropriate BMP can include: treatment required; cost (capital); maintenance restrictions; safety (motorist, GDOT maintenance staff, and general public); and site constraints (available right-of-way, soils, bedrock and water table, topography, setback requirements, and environmentally or socially sensitive areas). Start the feasibility evaluation with any infiltration BMPs included on the list of appropriate BMPs. Assess suitability of proposed topography to meet design criteria of BMPs. Altering proposed topography for the purposes of installing a BMP should also be considered (ex. regrading ditch or side slope) at this point, with the resulting effects to be assessed during the infeasibility assessment stage (i.e. additional earthwork costs, right-of-way impacts, etc.).

Note: depending on the unified sizing criteria applicable to the drainage basin, BMPs do not necessarily need to be placed at the outfall of each drainage basin. Because GDOT calculates water quality volume based on new impervious area, a BMP can be placed upstream of the outfall. The runoff that the BMP receives does not need to come from the new impervious area; the volume of runoff the BMP receives just needs to be equal to or greater than the water quality volume calculated from the new impervious area. Therefore, if claiming a BMP is not appropriate because the drainage area is larger than the allowable drainage area size listed in Chapter 10 of the Drainage Manual, additional explanation should be provided to indicate that the BMP cannot be placed either offline or upstream of the outfall where it would receive runoff from only a portion of the contributing drainage area.

For clarity, refrain from using the word “infeasible” or “not feasible” in this section when concluding if the BMP is appropriate or not for the basin with respect to fulfilling design constraints and requirements. Appropriate BMPs in this selection phase are carried forward and infeasibility is assessed in the next stage based on GDOT’s ten Infeasibility Criteria. Assessment of alternate construction methods (ex. constructing the BMP within cast-in-place walls, condensing the typical BMP/treatment train footprint, etc.) should also be included when assessing infeasibility if the alternate method will allow for a feasible BMP.

Filter Strip – The typical section for this section of the project in this area has curb and gutter. No sheet flow from impervious areas is present within the drainage limits. Therefore, the filter strip is not an appropriate BMP for this basin.

Grass Channel – A second BMP or oversizing of this BMP would be necessary to meet the required runoff reduction or 80% total suspended solids (TSS) removal. The grass channel is not an appropriate BMP since the other appropriate BMPs it could be combined with for this drainage area already have at least 80% TSS removal.

Infiltration Trench – The infiltration trench is an appropriate BMP for this basin. However, due to the fact that the enhanced swale will provide the required TSS removal at a lower cost, the infiltration trench will not be considered for this basin.

Sand Filter – The sand filter is an appropriate BMP for this basin.

Bioretention Area – The bioretention area is an appropriate BMP for this basin.

Dry Detention Basin – The dry detention basin is not an appropriate BMP on it’s own as it does not provide 80% TSS removal or RRv. The dry detention basin is not appropriate as part of a treatment train for this basin since the enhanced dry swale, the bioretention basin, or the sand filter would provide the WQv or the RRv with a smaller footprint.

Wet Detention Pond – The drainage area is less than 10 acres. Therefore, the wet detention pond is not an appropriate BMP for this basin.

Stormwater Wetland – The drainage basin is less than 5 acres. Therefore, the stormwater wetland is not an appropriate BMP for this basin.

Bioslope – The typical section for this portion of the project has curb and gutter. No sheet flow from impervious areas is present within the drainage limits. Therefore, the bioslope is not an appropriate BMP for this basin.

Enhanced Dry Swale – The enhanced dry swale is an appropriate BMP for this basin.

Enhanced Wet Swale – A permanent pool cannot be maintained. Therefore, the enhanced wet swale is not an appropriate BMP for this basin.

Open Graded Friction Course – OGFC is not present in this drainage area.

Regenerative Stormwater Conveyance – The construction costs for regenerative stormwater conveyances are high, so regenerative stormwater conveyance was not considered for this outfall since the considered BMPs are infeasible due to cost infeasibility.

Feasibility

For each BMP that was deemed appropriate for the drainage basin based on meeting design constraints and requirements, assess the infeasibility per GDOT’s ten Infeasibility Criteria. Provide BMP sizing calculations, Infeasibility Criterion #1 cost breakdowns, infeasibility displays, etc. in the appendices of the report to justify the infeasibility claim(s).

Enhanced Dry Swale – The enhanced dry swale with a capped underdrain was first evaluated. 2:1 slopes begin at the shoulder breakpoint and continue to the right-of-way. Any attempt to construct the enhanced dry swale over the required length will result in construction outside of the right-of-way. An enhanced dry swale with an open underdrain was then evaluated. It would also result in construction outside of the right-of-way. This BMP is infeasible under Infeasibility Criteria #2. Construction of the BMP would result in a delay to the project schedule greater than 90 days since no other right-of-way will be acquired for the project. See Appendix J for backup documentation.

Bioretention Area – The bioretention area with a capped underdrain was first evaluated. It will be constructed using cast in place walls. The cost of the bioretention area and minor earthwork required for construction was found to be 14.2% the cost of the roadway construction in this drainage basin. The cost of the bioretention area with an open underdrain would also exceed 10% of the roadway construction cost in the drainage basin. Therefore, this BMP is infeasible under Infeasibility Criteria #1. See Appendix J for cost calculations.

Sand Filter – The sand filter and sedimentation basin will be constructed using cast in place walls. The cost of the sand filter and minor earthwork required for construction was found to be 16.1% the cost of the roadway construction in this drainage basin. This BMP is infeasible under Infeasibility Criteria #1. The cost of the BMP will exceed 10% of the roadway construction cost in the drainage basin, the threshold established in the permit. See Appendix J for cost calculations.

Drainage Basin 1 was found to be infeasible for all BMPs.

For a BMP that was deemed feasible, provide BMP sizing calculations, cost breakdown, and construction plan sheets in the appendices of the report.

Drainage Basin 2 (Note: change basin name/number to correspond with Attachment B)

Include a brief description of the drainage basin location and where it discharges.

Drainage Basin 2 is located [from station 1+00 to station 2+00]. This drainage basin discharges [into a proposed 18-inch reinforced concrete pipe median drain that discharges inside of the right-of-way into a ditch before leaving the right-of-way].

Physical Parameters of Drainage Basin 2

Provide the pre- and post-development impervious and other land use of the basin used in the calculation of the flows, using the NRCS TR-55 Method. Include the hydrologic soil group and the composite curve number (CN) in the summary tables. Provide program input and output used to calculate the peak discharges in an appendix of the report. The land use tables should be for the total area to the outfall, unless detention is proposed. If detention is proposed then the post land use table should be separated into onsite and offsite areas.

| Drainage Basin 2 (Pre) | Area (ac) | CN |
| --- | --- | --- |
| Impervious | 0.49 | 98 |
| Woods - Good condition (Soil Group B) | 0.07 | 55 |
| Total | 0.56 | 93 |

| Drainage Basin 2 (Post) | Area (ac) | CN |
| --- | --- | --- |
| Impervious | 0.49 | 98 |
| Woods - Good condition (Soil Group B) | 0.07 | 55 |
| Total | 0.56 | 93 |

The peak flow table should be for the total area to the outfall.

|  | 1-Year (cfs) | 25-Year (cfs) | 100-Year (cfs) |
| --- | --- | --- | --- |
| Pre-Development | 1.84 | 2.69 | 3.89 |
| Post-Development | 1.84 | 2.69 | 3.89 |
| Change (Post - Pre) | 0.00 | 0.00 | 0.00 |
| Percent Change | 0.00% | 0.00% | 0.00% |

If an outfall level exclusion applies, specify which outfall level exclusion and provide the appropriate backup documentation as listed in the MS4 PCSR Help File.

There is no increase in impervious area in Drainage Basin 2. Therefore, Outfall Level Exclusion #6 will be used to eliminate the MS4 best management practice (BMP) for this basin. As stated in Section 4.2.5.1(a) of the GDOT MS4 permit, for outfalls along linear roadway projects whereby the net impervious surface area within that outfall’s drainage area has been reduced or remains the same as pre-developed conditions, post-construction stormwater requirements will not apply.

Downstream Analysis

Because this drainage basin uses OLE #6 with less than 0.1 acres of net new impervious area and post total drainage area remained within 0.1 acre of the pre total drainage area, a downstream analysis was not performed for Drainage Basin 2.

Drainage Area 3

Drainage Area 3 is located along [road name] between station [2+00] and station [5+00]. This drainage area discharges [a short distance across a grass field within a ditch then directly into an existing 15-inch pipe under a parking lot of a business].

Physical Parameters of Drainage Area 3

Provide the pre- and post-development impervious and other land use of the basin used in the calculation of the flows, using the NRCS TR-55 Method. Include the hydrologic soil group and the composite curve number (CN) in the summary tables. Provide program input and output used to calculate the peak discharges in an appendix of the report. The land use tables should be for the total area to the outfall, unless detention is proposed. If detention is proposed then the post land use table should be separated into onsite and offsite areas as shown for Drainage Area 3.

| Drainage Area 3 (Pre) | Area (ac) | CN |
| --- | --- | --- |
| Open space - Good condition (grass cover > 75%) (Soil Group B) | 1.25 | 61 |
| Open space - Good condition (grass cover > 75%) (Soil Group C) | 0.13 | 74 |
| Impervious | 0.35 | 98 |
| Woods - Good condition (Soil Group B) | 0.42 | 55 |
| Pre Total | 2.15 | 67 |

| Drainage Area 3 (Post Onsite) | Area (ac) | CN |
| --- | --- | --- |
| Open space – Good condition (grass cover > 75%) (Soil Group C) | 0.13 | 74 |
| Impervious | 1.25 | 98 |
| Post Onsite Total | 1.38 | 96 |

| Drainage Area 3 (Post Offsite) | Area (ac) | CN |
| --- | --- | --- |
| Open space – Good condition (grass cover > 75%) (Soil Group B) | 1.25 | 61 |
| Post Offsite Total | 1.25 | 61 |

| Post Total | 2.63 | 79 |
| --- | --- | --- |

The peak flow table should be for the total area to the outfall.

| Total Flows to Outfall | 1-Year (cfs) | 25-Year (cfs) | 100-Year (cfs) |
| --- | --- | --- | --- |
| Pre-Development | 2.20 | 6.21 | 9.74 |
| Post-Development (unattenuated) | 9.45 | 15.40 | 21.85 |
| Change (Post - Pre) | 7.25 | 9.20 | 12.11 |
| Percent Change | 329.55% | 148.15% | 124.33% |

If an outfall level exclusion does not apply, this section is used to discuss the feasibility of installing a BMP in the drainage basin. First, evaluate the applicable unified sizing criteria volumes. The data in this table should be for the onsite area.

Water Quality and Channel Protection

| Total Drainage Area (ac) | 2.63 |
| --- | --- |
| Pre-Developed Impervious Area (ac) | 0.35 |
| Post-Developed Impervious Area (ac) | 1.25 |
| Pre-Developed % Impervious | 13.31 |
| Post-Developed % Impervious | 47.53 |
| Runoff Coefficient (Rv) | 0.308 |
| Target RRv (ft3) | 2,940 |
| Target WQv (ft3) | 3,528 |
| Target CPv (ft3) | 9,619 |

Supporting stormwater runoff quality/reduction (runoff reduction volume and water quality volume) and channel protection volume calculations are included in Appendix E and Appendix F, respectively.

Downstream Analysis

A downstream analysis was performed for Drainage Area 3. The downstream study point establishes a drainage area that is approximately 10 times as large as the on-site drainage area. See Appendix H for a map showing the drainage areas and downstream study point.

The USGS StreamStats website was used to determine the downstream analysis drainage area along with knowledge of existing crossdrains present in the drainage area. The following table shows the physical parameters of the downstream analysis drainage area, not including the on-site drainage area. As the on-site drainage area is not included, the physical parameters will be the same for pre- and post-developed conditions.

| Drainage Area 3 Downstream Analysis (Minus On-site) | Area (ac) | CN |
| --- | --- | --- |
| Commercial and business (Soil Group B) | 17.22 | 92 |
| Woods - grass combination - Good condition (Soil Group B) | 3.96 | 58 |
| Total | 21.18 | 86 |

The longest flow path and the average area slope were obtained from the USGS topographic map and used to determine the time of concentration using the lag method.

| Drainage Area 3 Downstream Analysis |  |  |
| --- | --- | --- |
| L | 1486.3 | Flow Length (ft) |
| CN | 86 | Curve Number |
| Y | 9.8 | Watershed Slope (%) |
| S | 1.63 | Maximum Retention (in) |
| Tc | 0.19 | Hours |
| Tc | 11.4 | Minutes |

The channel routing function in Hydraflow Hydrographs was then used to model the timing of the hydrographs. See Appendix G for copies of the hydrographs.

|  | Pre-Development | Without BMP |  |  | With BMP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Post-Development | Change |  | Post-Development | Change |  |
|  |  |  | (cfs) | (%) |  | (cfs) | (%) |
| 25-Year Peak Flow (cfs) | 118.78 | 123.76 | 4.98 | 4.19% | 121.56 | 2.78 | 2.34% |
| 100-Year Peak Flow (cfs) | 154.58 | 159.74 | 5.16 | 3.34% | 157.70 | 3.12 | 2.02% |

Warranting Criteria

Detention is recommended for drainage area 3 based on warranting criteria #3 “Post-developed flows without detention would flood downstream property”.

Discharge from the outfall goes a short distance across a grass field within a ditch then directly into an existing 15-inch pipe under a parking lot of a business. The parking lot slopes down to the business so any overtopping of the parking lot could cause flooding of the business.

Survey data for the 15-inch pipe and the top of the parking lot has been collected. Flows have been modeled in HY-8. In the pre-developed condition, the peak flows for the 25-year storm safely pass through the 15-inch pipe without overtopping the parking lot. However, in the post-developed unattenuated condition the 25-year storm would overtop the parking lot, posing a flooding concern to the business. See Appendix H for HY-8 modeling documentation.

The right-of-way team has met with the owners of the property and discussed increasing the size of the 15-inch pipe or installing an additional pipe under the parking lot. However, the property owner stated that this is not a possibility. See Appendix H for the right-of-way team’s notes on these meetings.

As demonstrated by the topography of the location maps, the discharge cannot be bypassed around the parking lot while maintaining gravity flow. Therefore, the design team recommends installation of a post-construction BMP in the grass field between the state route and the parking lot. The pond model in Appendix G in conjunction with the HY-8 model in Appendix H demonstrate that the recommended BMP would reduce peak flows so that the 25-year storm both safely pass through the 15-inch pipe without overtopping the parking lot. Attenuated post-developed peak flow for the 100-year storm is less than the 100-year pre-developed peak flow.

Best Management Practice (BMP) Selection

Filter Strip – The typical section for this section of the project in this area has curb and gutter. No sheet flow from impervious areas is present within the drainage limits. Therefore, the filter strip is not an appropriate BMP for this basin.

Grass Channel – The grass channel is not an appropriate BMP for this basin due to the high groundwater table.

Infiltration Trench – The infiltration trench is not an appropriate BMP for this basin due to the high groundwater table.

Sand Filter – The sand filter is not an appropriate BMP for this basin due to the high groundwater table.

Bioretention Area – The bioretention area is an appropriate BMP for this basin.

Dry Detention Basin – The dry detention basin is not an appropriate BMP due to the high groundwater table.

Wet Detention Pond – The drainage area is less than 10 acres, however groundwater is present so this BMP is appropriate for this basin.

Stormwater Wetland – The drainage basin is less than 5 acres. Therefore, the stormwater wetland is not an appropriate BMP for this basin.

Bioslope – The typical section for this portion of the project has curb and gutter. No sheet flow from impervious areas is present within the drainage limits. Therefore, the bioslope is not an appropriate BMP for this basin.

Enhanced Dry Swale – The enhanced dry swale is not an appropriate BMP for this basin due to the high groundwater table.

Enhanced Wet Swale – The enhanced wet swale is not an appropriate BMP for this basin since detention is recommended to be warranted for this basin and the enhanced wet swale provides minimal detention.

Open Graded Friction Course – OGFC is not present in this drainage area.

Regenerative Stormwater Conveyance – The construction costs for regenerative stormwater conveyances are high, so regenerative stormwater conveyance was not considered for this outfall since the considered BMPs are infeasible due to cost infeasibility.

Feasibility

Wet Detention Basin – The wet detention basin would require purchase of expensive ROW. The cost of the wet detention basin (including ROW) required for construction was found to be 16.7% the cost of the roadway construction in this drainage basin. This BMP is infeasible under Infeasibility Criteria #1. The cost of the BMP will exceed 10% of the roadway construction cost in the drainage basin, the threshold established in the permit. See Appendix J for cost calculations.

Drainage Basin 3 was found to be infeasible for all BMPs. However, the wet detention basin will be assessed for suitability since detention is recommended to be warranted.

Suitability

Check each box to certify that the proposed BMP is suitable.

☒ The project would not require an existing roadway alignment change or a safety concern solely to allow for BMPs.

☒ The installation of post-construction BMPs would not require the re-alignment and/or piping of a stream.

☒ Implementation of BMPs would not cause loss of habitat for endangered or threatened species.

☒ Implementation of the BMP would not cause significant damage to cultural or community resource such as an historical site, archeological site, cemetery, park, wildlife refuge, nature trail, or school facilities.

☒ Implementation of BMPs would not result in violation of state or federal law or regulation.

☒ There are no foreseen site limitations including: shallow bedrock, contaminated soils, high groundwater, utilities, or underground facilities if avoidance or relocation is not suitable.

☒ Site does allow for gravity flow to the appropriate BMP.

Drainage Area 3 was found to be suitable for a wet detention basin.

Appendix A – Site Location and Drainage Basin Maps

Include maps showing the overall project location as well as the pre- and post-development drainage basins.

Appendix B – NOAA Precipitation Table

Include a printout of the NOAA precipitation table for the midpoint of the project (hdsc.nws.noaa.gov/hdsc/pfds/pfds_map_cont.html?bkmrk=ga).

Appendix C – Soils Map

Include soil map(s) showing the hydrologic soil group in each drainage basin.

Appendix D – Environmental Resource Impact Table

Include a copy of the Environmental Resource Impact Table (ERIT).

Appendix E – Stormwater Runoff Quality / Reduction Calculations

Include runoff reduction and water quality volume calculations for those drainage basins that do not have an outfall level exclusion.

Appendix F - Channel Protection Calculations

Include channel protection volume calculations for those drainage basins where channel protection is applicable.

Appendix G - Hydrologic Model Output

Provide the hydrologic model output for each drainage basin. Only include the output for the 1-year, 25-year, and 100-year 24-hour storms.

Appendix H - Downstream Analysis Documentation

Provide the backup documentation supporting the downstream analysis for each drainage basin unless OLE #6 based on minimal net new impervious area applies and total post drainage area remains within 0.1 acre of the total pre drainage area (unless the basin is included in the downstream analysis for another basin). A downstream analysis is not required if the drainage basin discharges directly to a channel or waterbody that has a drainage area larger than five square miles.

Provide:

- Drainage basin maps and aerial photography defining existing and proposed drainage basins indicating topography and flow paths

- Field notes, photographs, and any correspondence with local residents or other contacts

- Flood maps (if available)

- Hydrologic and hydraulic calculations and model inputs/outputs

Appendix I – Outfall Level Exclusion Documentation

Provide backup documentation supporting each outfall level exclusion claimed. See the MS4 PCSR Help File for specific information that needs to be submitted for each outfall level exclusion.

Appendix J – Infeasibility Documentation

Provide backup documentation supporting each infeasibility claimed. See the MS4 PCSR Help File for specific information that needs to be submitted for each infeasibility.

Appendix K – Stormwater BMP Infiltration Worksheet(s)

Stormwater BMP Infiltration Worksheet(s) is required when the project considers installating an infiltration BMP. Submit the Stormwater BMP Infiltration Worksheet(s) to ODPS. Submit the GDOT-approved Stormwater BMP Infiltration Worksheet(s) with this MS4 Post-Construction Stormwater Report. See Appendix J of the Drainage Manual for additional guidance.

Appendix L – BMP Design Calculations

Provide the design calculations for feasible BMPs.

Appendix M – Feasible BMP Cost Calculations

Provide a cost estimate for each feasible BMP. If detention is warranted, provide a cost estimate for each detention BMP that is warranted and suitable.

Appendix N – Construction Plan Sheets

Provide a copy of the entire current plan set.

Attachment D

Milestone Plan Submittal Checklist

Preliminary Field Plan Review (PFPR) Milestone (or Costing Plans)

Yes / No

☐    ☐  Has the concept hydrology study been altered?

☐  A detailed study has been provided including the design of detention and water quality structures

☐  The detailed design includes all of the following:

☐ Percent impervious

☐ Drainage area

☐ Runoff (C) or (CN) values

☐ Average slope of site

☐ Soil conditions

☐ Stage/Storage/Discharge Table

☐ Grading necessary for any BMPs

☐ Time of concentration

Yes / No

☐    ☐  The Post-Construction BMP Summary Table and Pre- versus Post-Development Drainage Area Summary Table have been completed.

☐    ☐  (For infiltration BMPs) A Stormwater BMP Infiltration Worksheet(s) has been completed and approved by GDOT.

☐    ☐  The Low Impact Development (LID) / Green Infrastructure (GI) Checklist been completed.

☐    ☐  The Post-Construction Stormwater BMP Documentation has been completed.

☐   ☐  Has a discussion with those responsible for BMP maintenance (GDOT, local government, etc.) been    completed and documented?