# 2. Limited Scope Concept Report Template

## A-2.1	Concept Reports for “Limited Scope” Reports

Projects having a limited scope may use an abbreviated Concept Report format. Limited Scope Concept Reports (LSCR) fall into two categories: Type I and Type II. Projects that qualify to use the LSCR Type I abbreviated format should meet the following conditions:

- Exempt federal oversight status (if federally funded) or locally/state funded. PoDI projects may be eligible if prior consent is obtained from FHWA or if FHWA has not selected the Concept for involvement in the PoDI Stewardship and Oversight (S&O) plan for the individual project.

- Limited environmental impacts (GEPA or NEPA Categorical Exclusion anticipated)

- No or only minor ROW requirements (e.g., few parcels impacted, no major impacts to individual parcels, few displacements anticipated)

- No VE study requirement

- No PAR required

- Traffic Management Plan requires only TTC, if applicable

- No or only limited utility impacts

The LSCR Type II format is similar to the Type I format and would also follow Appendix A-2 but would not require an alternatives analysis. Projects that qualify to use the Limited Scope Type II (formerly referred to as “Directed Scope Concept Report”) abbreviated format should generally meet the following conditions:

- Any type of non-federal aid project (regardless of right-of-way requirements/impacts)

- Prior approval from the Chief Engineer

- Projects with a completed PAR, IMR or IJR may also qualify for using the LSCR Type II format

If any of the above requirements/qualifications are not met, the full Project Concept Report format (Appendix A) should be utilized.  Exceptions may be granted by the State Design Policy Engineer on a case-by-case basis.

The project team should keep the limited project scope in mind when considering the level of study required. For example, passing lane and bridge replacement projects do not normally require the preparation of completed capacity or safety studies for inclusion in the concept report. However, an appropriate level of evaluation is required to identify safety and operational needs. The Concept Scoping Tool located on R.O.A.D.S. under Plan Development Process > Concept Scoping Tool provides guidance on the level of work anticipated for various project types to assist with developing the project scope of work.

The Limited Scope Concept Report template is intended to provide basic guidance for a wide variety of project types. The project team is encouraged to use their best judgement and modify the Limited Scope Concept Report template as appropriate for specific projects.

## A-2.2	Limited Scope Concept Report Template

Please refer to Template A, section A.3 General Instruction and Information for more information. If questions during development of the concept report, please contact the Office of Design Policy and Support’s Conceptual Design Group at ConceptReports@dot.ga.gov for assistance.

- Refer to the Concept Scoping Tool which defines the minimum concept report scope required to be addressed for thirteen project categories. This tool is located on the GDOT ROADS web page on the “View Manuals & Guidelines” page in the “Plan Development Process” category.

- Please use the most current version of the Concept Report when submitting your report. An up-to-date MS-Word template is available for download on the R.O.A.D.S. Manuals & Guides web page and/or may be provided by the Office of Design Policy and Support’s Conceptual Design Group for your use upon request.

- Hints & Tips

  - There are a number of pull-down menus and check boxes available in the MS-Word versions of the Concept Reports.

  - Instructions and information to assist in completing the report are shown in blue italics for easy identification and should be removed prior to report submission.

  - Some of the formatting is set up in tables with invisible borders – turning on View Table Gridlines will make these easier to see.

  - Make sure all attachments, maps, layouts, etc. are clear, legible and rotated appropriately.

  - An additional layout in the form of a PIOH style overview display (large format) may be submitted separately for lengthy or large project concepts.

- Reports should be submitted in PDF format by placing the report in ProjectWise and emailing a link to the report to: ConceptReports@dot.ga.gov

- Design Variances and Design Exceptions - Please note that FHWA typically requires that Design Variances and Design Exceptions be approved prior to approval of the Concept Report for selected PoDI projects; for other projects, Design Variances and Design Exceptions are normally identified during concept phase and formally requested during the Preliminary Design Phase.

## A-2.3	Concept Report Template

If you have any questions regarding the Concept Report, please contact the Office of Design Policy and Support at ConceptReports@dot.ga.gov

See following pages.

|  |  | Limited Scope Concept Report Select Type Template version: 2025.08.08 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project Type: |  |  |  | P.I. Number: |  |
| GDOT District: |  |  | County: |  |  |
| Federal Route Number: |  |  | State Route Number: |  |  |
| Brief Project Description - should be no more than 2-3 lines long |  |  |  |  |  |

Submitted for approval:  Remove ALL blue guidance & delete any inapplicable signature lines

| Consultant Designer & Firm or GDOT Design Phase Office Head & Office (edit as appropriate) |  | Date |
| --- | --- | --- |
| Local Government Sponsor (edit if applicable, remove if not locally sponsored) |  | Date |
| Community Improvement District (CID) Sponsor (Remove if not CID sponsored) |  | Date |
| State Program Delivery Administrator |  | Date |
| GDOT Project Manager |  | Date |

# Recommendation for approval: Remove ALL blue guidance & delete any inapplicable signature lines.

| State Environmental Administrator |  |  | Date |
| --- | --- | --- | --- |
| State Traffic Engineer |  |  | Date |
| District Engineer |  |  | Date |
| State Bridge Engineer (if applicable; remove if no structures are included in project) |  |  | Date |
| ☐ | MPO Area: This project is consistent with the MPO adopted Regional Transportation Plan (RTP)/Long Range Transportation Plan (LRTP). |  |  |
| ☐ | Rural Area: This project is consistent with the goals outlined in the Statewide Transportation Plan (SWTP) and/or is included in the State Transportation Improvement Program (STIP). |  |  |
| for Division of Planning |  |  | Date |

Approval:

| Concur: |  |  |  |
| --- | --- | --- | --- |
|  | GDOT Director of Engineering |  | Date |
| Approve: |  |  |  |
|  | GDOT Chief Engineer |  | Date |

PROJECT LOCATION MAP

Add a project location map image that is sharp, clear and legible which sufficiently locates the project.  Please include the following information:

- Project Name (Typically from PSR unless project limits require updating)

- Nearby cities/towns and approximate distance from project

- North arrow

- Label the approximate project beginning and ending points along mainline or identify project site for intersection improvement or similar scale project

- Label any tie-ins to adjacent projects and/or project phases

- Roadway name(s) and/or other information useful in easily locating the project (e.g., nearby waterways, lakes, towns, other well-known sites, etc.)

PLANNING & BACKGROUND DATA

Project Justification Statement was Prepared (or Approved) By: (GDOT Office)	on:   Date

Project Justification Summary:  A brief statement summarizing the major issue(s) that the project is intended to address, derived from the Project Justification Statement typically provided by or approved by the Office of Planning, the Office of Bridge Design and Maintenance, or the Office of Traffic Operations. The Project Justification should include:

- Any designated programs or highway networks that the project is included in (e.g., GRIP, STRAHNET, Oversized Truck Route, designated bike route, APD, etc.).

- How the project originated - for example: Transportation Board, Senior Management, Planning Office, planning study, local government, MPO, Operations, Bridge Maintenance, etc.

- A summary of the major issue(s) to be addressed by the project. For example: pedestrian mobility, congestion/LOS/capacity issues, high crash rates, geometric or structural issues, legislative program requirements (e.g., GRIP), infrastructure improvements, streetscapes, etc. Briefly include supporting data such as current or projected LOS, crash history, etc.

- Other relevant information regarding the issue(s) the project is intended to address.

- Performance goals – in general, what is the major performance goal of the project (e.g., reduce congestion, improve mobility, reduce crashes, correct geometric and/or structural deficiencies, etc.).  Also list any expected secondary benefits the project is expected to provide.

The Project Justification should only include information relevant to the issue(s) to be addressed. Please do not describe possible solutions or include other detailed information. If the full Project Justification Statement document is longer than one page, please summarize as indicated above and attach the full Project Justification Statement to the concept report.

Note: The Design Phase Leader will update the Project Justification as needed, and the originating or approving office will review it as part of the concept report approval process.

Existing conditions: A brief general description of the project location as it currently exists such as intersection control, number of lanes, shoulder widths, medians, sidewalks, shared use paths, bicycle lanes, major intersections, substandard skew angles, structures, major utilities in project area, etc.

Other projects in the area:  List projects in the area (e.g., GDOT, local agency and significant private development projects) that may impact or be impacted by this project; include PI numbers (if applicable), current project phase, and a brief description of any project coordination needed.

MPO:  MPO Name  if applicable		TIP #: if applicable

Congressional District(s):  District

Federal Oversight:	☐ PoDI	☐ Exempt	☐ State Funded	☐ Other

If this is an FHWA Project of Division Interest (PoDI), check the S&O Plan to verify whether FHWA wishes to review & approve the Concept Report.

Projected Traffic:  	24 HR T:       %	Current Year (20ww):

Open Year (20xx):       	Design Year (20yy):

Traffic data source: (e.g., TADA or field counts)

Traffic projections performed by:   GDOT Office or Design Firm

Traffic projections approved by: GDOT Office		 Date approved:    Date

Include data for the mainline and for any major roadways intersecting the project. For intersection projects, include data for each leg. Concept level traffic data may be obtained from the GDOT Traffic Analysis & Data Application (TADA) web page where turning movement analysis is not necessary and truck data for pavement design is available on TADA.

AASHTO Functional Classification:  Functional Classification

Refer to the GDOT Functional Classification Map and Section 3.1 of the GDOT DPM.

AASHTO Context Classification (Mainline):  Context Classification

See Section 1.5 of AASHTO’s 7th Ed., A Policy on Geometric Design of Highways and Streets (“The Green Book”)

AASHTO Project Type:  Project Type

Refer to Section 1.7 of the AASHTO Green Book. See drop down lists for selections.

Is the project located on an NHS roadway? 	☐ No 	☐ Yes

Refer to the GDOT Functional Classification Map web page.

Complete Streets - Bicycle, Pedestrian, and/or Transit Standards Warrants:

Warrants met:	 ☐ None	 ☐ Bicycle	☐ Pedestrian	☐ Transit

Public bus stop impacts anticipated:  ☐ None    ☐ Temporary 	☐ Relocation(s)    ☐ Undetermined

Check all boxes that apply and then note which Standard Warrants from Section 9.4 of the GDOT Design Policy Manual are met (e.g., Pedestrian Warrant #2, Bicycle Warrant #3). For each box checked, briefly explain how each Standard Warrant is met. Include State Bicycle Network route number(s) if applicable. Attach summary of any Bicycle, Pedestrian, and Transit Warrant Studies completed, or summarize results here.  See Section 9.4 of the GDOT Design Policy Manual for further guidance. If municipal/public bus stops are along the project, indicate if they would be impacted during construction or if they would be relocated or rebuilt. Additional coordination and design may be needed, along with Public Information efforts – Please contact GDOT NEPA if there may be bus stop impacts.

Is this a 3R (Resurfacing, Restoration, & Rehabilitation) Project?	☐ No 		☐ Yes

See section 11.1 of the GDOT Design Policy Manual for 3R standards.

Pavement Evaluation and Recommendations

Initial Pavement Evaluation Summary Report Required? 		☐ No 		☐ Yes

Feasible Pavement Alternatives:  		☐ HMA	☐ PCC	 	☐ HMA & PCC

[HMA = Hot Mix Asphalt; PCC = Portland Cement Concrete] The Initial Pavement Evaluation Summary Report, if required, should be completed prior to submission of the Concept Report for approval. The Office of Materials and Testing would prepare this report upon request.  The pavement report should be attached to the Concept Report. Please reference Chapter 5 of the PDP and the Pavement Design Process Flowchart for further information. Final pavement design approval occurs during the Preliminary Design Phase.

Is the project located on a Special Roadway or Network? ☐ No   ☐ Yes - Network

If no, delete the Network pull-down box. If yes, list all networks that apply (a dropdown list is provided above). See GDOT Design Policy Manual section 3.2.3 for additional design vehicle requirements to apply.

Do the limits of the project include one or more signalized intersections?	☐ No	☐ Yes

The signal group in the Office of Traffic Operations (SigOps) will help coordinate the operation of traffic signals throughout the development and construction of this project and help ensure that the correct jurisdictions are involved. Contact the SigOps team to notify them of upcoming projects by emailing SigOps@dot.ga.gov

Is Federal Aviation Administration coordination anticipated?  ☐ No	☐ Yes

Some construction activities may require FAA coordination if located close to a public use airport or navigation site. This should be discussed at the project Concept Team Meeting. For more information, contact AviationPrograms@dot.ga.gov and utilize FAA’s Notice Criteria Tool, available at: FAA Notice Criteria Tool web link

Are any GDNR boat ramps located within the project limits?   ☐ No   	☐ Yes

Boat ramps managed by the Georgia Department of Natural Resources (GDNR) require additional coordination in compliance with an interagency agreement signed by GDOT and GDNR on March 30, 2020. If GDNR boat ramps are within the project limits, the Environmental Survey Boundary (ESB) in the vicinity of the boat ramps should be large enough to accommodate relocation of the boat ramps.  Locations of GDNR boat ramps may be found at: https://experience.arcgis.com/experience/711be0965b0e45b39cf71b0288521620/

DESIGN AND STRUCTURAL

Description of the proposed project: A brief general description of the major construction elements. Include the approximate proposed length, and general location of the project, any city and county limits or proximity thereto.  Specific design criteria (e.g., lane width, design speed, etc.) should be kept to a minimum, since they will be described in a later section.  Identify and describe any context sensitive and/or practical design solutions to be utilized on the project. If it is an ITS Project, summarize the Concept of Operations briefly. More detailed information on structures should be included in the table below.

Major Structures: If there are no major structures on project, state “N/A” and delete table below.

| Structure | Existing | Proposed |
| --- | --- | --- |
| Bridge/structure ID # and/or Location | Describe existing structure - length & width, typical section, including lane and shoulder widths, etc. of existing structure. | Describe proposed structure length & width, typical section including lane and shoulder widths, etc. of proposed structure (bridge typical section should be in attachments). |
| Retaining walls (not including gravity walls) | Describe existing structure | Describe proposed structures |
| Other | Describe existing structure Length and span x height | Describe proposed structures Length and span x height |

Accelerated Bridge Construction (ABC) techniques anticipated: 	 ☐ No	 ☐ Yes

Briefly explain why ABC techniques are/are not anticipated. If ABC techniques are anticipated, provide a brief description of the possible ABC techniques, an estimate of construction duration, an estimate of road closure duration, and the appropriate tier of ABC. For additional guidance, see the GDOT Bridge Office webpage. If no bridge construction is anticipated for the project, please delete this section.

Mainline Design Features:

NOTE: GDOT, AASHTO and FHWA Design Standard criteria are listed in bold text. GDOT Guidelines are listed in standard text. Please use additional copies of the table below as needed for other major roads, significant side roads, intersection legs for intersection improvement projects, etc. Multiple roads with similar functional classification/characteristics/typical section may also be combined into a single table as warranted. For projects whose justification is not primarily driven by operational or safety needs (e.g., projects driven by legislation such as GRIP) with low traffic volumes, opportunities for DE/DVs should be routinely considered to reduce project costs and impacts except where substandard geometry is linked to a significant crash history.

| Roadway name(s) | Functional Classification: Functional Classification |  |  |
| --- | --- | --- | --- |
| Feature Standard criteria are listed in bold text | Existing | *Policy | Proposed |
| Typical Section: |  |  |  |
| - Number of Through Lanes |  |  |  |
| - Lane Width(s) (-ft) |  |  |  |
| - Median Width (-ft) & Type |  |  |  |
| - Shoulder Width (-ft) (Outside) see DPM Section 6.5 rural – include overall & paved widths |  |  |  |
| - Border Area Width (-ft) urban shoulder, see DPM Section 6.7 |  |  |  |
| - Cross Slope (%) see DPM Section 6.3 |  |  |  |
| - Outside Shoulder Slope (%) see DPM Section 6.5 |  |  |  |
| - Inside Shoulder Width (-ft) see DPM Section 6.5 include overall & paved widths |  |  |  |
| - Sidewalks (-ft) (width/type) see DPM Section 9.5.1 |  |  |  |
| - Auxiliary Lanes (# LTL, RTL or TWLTL / -ft width) |  |  |  |
| - Bike Accommodations see DPM Section 9.4.2 |  |  |  |
| Posted Speed (mph) |  |  |  |
| Design Speed (mph) see DPM Section 3.3 |  |  |  |
| Minimum Horizontal Curve Radius (-ft) see GDOT DPM Section 4.2 |  |  | Min radius anticipated |
| Maximum Superelevation Rate (%) see GDOT DPM Section 4.5, Table 4.8 |  |  |  |
| Maximum Grade (%) see GDOT DPM Section 4.3 |  |  | Max design grade anticipated |
| Access Control see GDOT DPM Section 3.5 |  |  |  |
| Design Vehicle see GDOT DPM Section 3.2 |  |  |  |
| Check Vehicle see GDOT DPM Section 3.2 |  |  |  |
| Pavement Type see GDOT DPM Sections 6.2 and 10.4 |  |  |  |
| Additional rows / items can be included as warranted; listed features above can be deleted if not relevant/applicable |  |  |  |

*According to current GDOT Design Policy if applicable

Roundabout Design Features:  If no roundabouts are included in project, please delete this section/table.

|  | RBDG Guideline | Roadway #1 Name | Roadway #2 Name |
| --- | --- | --- | --- |
| Circulating Lane Width (ft) | (Typically 14/18/20ft) |  |  |
| Circulatory Design Speed (mph) | (Typically 15/25mph) |  |  |
| Design Vehicle | (Typically WB-67 on SR; see RBDG table 3-1) |  |  |
| Check Vehicle | (email OSOW@dot.ga.gov) |  |  |
| Pedestrian Accommodations | (ADA and crosswalk (min)/ + sidewalk) |  |  |
| Bicycle Accommodation | (8-10ft SUP/shared lane) |  |  |
| Splitter Island Length (ft) |  |  |  |
| Splitter Island Width (ft) |  |  |  |

See the Roundabout Design Guide (RBDG) on the R.O.A.D.S. website for further information.

Design Exceptions/Design Variances to FHWA or GDOT Controlling Criteria anticipated:

See Chapter 2 of GDOT’s Design Policy Manual and the current Concept Report Template (Appendix A) on GDOT’s ROADS webpage for additional guidance. If no DEs or DVs are anticipated, state None; otherwise list here and describe Important DE/DVs should be discussed with the Roadway Policy Group to verify feasibility of approval prior to submission of the concept report, particularly where Design Controls or other safety-related criteria (e.g., stopping sight distance) are affected and failure to obtain approval of the DE/DV(s) would substantially impact the project.

Design Variances to GDOT Standard Criteria anticipated:

See Chapter 2 of GDOT’s Design Policy Manual and the current Concept Report Template (Appendix A) on GDOT’s R.O.A.D.S. webpage for additional guidance. State None If no DVs for GDOT Standard Criteria apply; list and describe any that are anticipated.

Lighting:   ☐ No   ☐ Yes

If lighting is proposed, describe the location and basic type (roadway, interchange, roundabout, pedestrian, tunnel, etc.). Briefly explain why lighting is proposed (i.e. AASHTO lighting warrants met, required by GDOT policy, mitigation for a design exception or variance, requested by local government, etc.) as applicable. A Lighting Support Letter should be obtained for all proposed lighting on state routes.

Off-site Detours Anticipated:   ☐ No   	☐ Undetermined   ☐ Yes

If yes: 	Roadway type to be closed:		☐ Local Road	☐ State Route

Detour route selected:	☐ Local Road	☐ State Route

District concurrence with detour route:	☐ No/Pending	☐ Received Date

Detour presented to public: 	☐ No	☐ Yes Date

Where off-site detours are to be utilized, documentation of early coordination with local emergency services, school board, the District Office, and other identified stakeholders should be included in attachments. More extensive coordination may be needed for detours with the potential for major impacts to stakeholders and local roads. A formal Detour Report will be required where roadway closures are anticipated to exceed 5 days in duration. For on-site detours, include a detour layout after the Concept Layout in attachments. Road User Cost calculations (https://highways.dot.gov/federal-lands/design/tools-wfl/road-user-cost-calculator) for off-site detours should be attached if the costs were a factor in the selection of the Preferred Alternative.

Transportation Management Plan [TMP] Required:   ☐ No   ☐ Yes  TMP Classification

As part of the federal Work Zone Safety and Mobility Rule, all Federal-aid highway projects require a TMP. Projects classified as “Non-Significant” may only require a Temporary Traffic Control (TTC) plan, often covered under Special Provision 150.  Projects classified as “Significant” require a complete TMP and formal TMP report which includes a TTC plan and addresses Transportation Operations (TO) and Public Information (PI) components.  If needed, the formal TMP report would typically be developed during the preliminary plans phase. For more information, see GDOT Policy 5240-1.

INTERCHANGES AND INTERSECTIONS

Interchanges/Major Intersections:  List and briefly describe any interchanges or major intersections along project.

Intersection Control Evaluation (ICE) Required:   ☐ No   ☐ Yes

Refer to GDOT’s ICE Policy for guidance.  Attach to this report where applicable:

- ICE Stage 1 – Screening Decision Record

- Signed Concurrence Memo or waiver

Completion of ICE Stage 1 is required for all projects where intersections are impacted. When required, ICE Stage 2 should not be included in the Concept Report but will be accepted for review and approval by OTO during early preliminary design at a Traffic Operations 30% Plans Review. The designer will still need to perform sufficient studies to support the intersection controls shown on the concept layout.

Roundabout Concept Validation Required:   ☐ No   ☐ Yes	   	☐ Completed  Date

Roundabout layout checks are required only for complex or highly constrained roundabouts. In other cases, they should not be included and would be submitted for review during early preliminary design at a Traffic Operations 30% Plans Review. Refer to Chapter 8.2.3 of GDOT’s Design Policy Manual for guidance. Note - The Office of Traffic Operations will require access to design files to complete their review.

UTILITY AND PROPERTY

Railroad Involvement: All projects located within 500 feet of railroad property are to be evaluated by the Office of Utilities – Railroad Liaison to determine if railroad coordination is required.  Note results and/or include correspondence in Attachments.  A cost estimate for RR coordination and RR construction/review should be attached, if applicable.  Consult the Railroad Liaison Manager and/or the Utility Railroad Crossing Manager in the Office of Utilities for Railroad coordination requirements. Discuss ownership and future use of the railroad (e.g., proposed new rail lines, freight or passenger rail, number of trains per day, pre-emption etc.).

Utility Involvements: List any identified utilities which may be impacted by project, including type and owner. Include SRTA/GRTA where appropriate.

SUE Required:   ☐ No   ☐Yes

Verify if SUE (Subsurface Utility Engineering) would be utilized on the project. By policy, SUE is required for all projects with a Commissioner approved Public Interest Determination Recommendation.

Public Interest Determination Policy and Procedure recommended:    ☐ No   ☐ Yes

See Policy and Procedures Subject Nos. 6863-12 and 3E-1 for guidance.  If yes, describe the Concept Team’s findings and recommendations. Attach Utility Risk Management Plan with Risk Acceptance or Risk Avoidance recommendations if applicable.

Right-of-Way (ROW): 	Existing width:       -ft.	Proposed width:       -ft.

Refer to Chapter 3 of GDOT’s Design Policy Manual for guidance. Check all easement types that apply. Parcels should match ROW Cost estimate summary and concept layout.

Required Right-of-Way anticipated:	☐  None	☐ Yes	☐ Undetermined

Easements anticipated: 	☐  None	☐ Temporary	☐ Permanent *	 ☐ Utility	☐ Other

* Permanent easements include the right to place utilities.

| Anticipated total number of impacted parcels: |  |  |
| --- | --- | --- |
| Displacements anticipated: | Businesses: |  |
|  | Residences: |  |
|  | Other: |  |
| Total Displacements: |  |  |

Location and Design approval:   ☐ Not Required   ☐ Required

Location and Design approval is needed for all projects where ROW or permanent easements are to be acquired.

Impacts to federally managed property anticipated:   ☐ No   ☐ Yes   ☐ Undetermined

Identify all federally owned or managed properties directly impacted by the project. If alterations to property rights from USACE managed properties are anticipated, additional coordination with the USACE will be required, possibly including a Real Estate Outgrant. For additional guidance, see the USACE Real Estate Outgrant Process Flowchart, available on GDOT’s external webpage under Business and Government > Design Manuals > Manuals & Guides > Plan Development Process > Category: Flowcharts > USACE Real Estate Outgrant Process.

ENVIRONMENTAL AND PERMITS

Anticipated Environmental Document:  Document Type

Select document type from pull-down menu.  ‘GEPA ~ None’ is for state-funded projects where total project cost is expected to be less than $100 million.

Level of Environmental Analysis – The environmental considerations are based on: (check one)

☐  A preliminary desktop or screening level environmental analysis and are subject to revision after the completion of resource identification, delineation, and agency concurrence.

☐  Completion of resource identification and delineation and are subject to revision after the completion of agency concurrence.

☐  Completion of resource identification, delineation, and agency concurrence.

GDOT MS4 Permit Compliance – Is the project located in a GDOT MS4 area?         ☐ No   ☐ Yes

If yes, is the GDOT MS4 Permit anticipated to apply to all or part of this project?   ☐ No   ☐ Yes

See MS4 Concept Report Summary, available at:  R.O.A.D.S.> Manuals and Guides> Roadway> Category: Stormwater Permit (MS4) & Special Design Post-Construction Details> MS4 Concept Report Summary [https://www.dot.ga.gov/GDOT/pages/DesignManualsGuides.aspx]. Evaluate for Project Level Exclusions (PLEs) 1-4 and 7-8. If one or more PLE 1-4 or 7-8, is applicable, fill out the PLE evaluation section of the MS4 Concept Report Summary and attach to the Concept Report. If no PLE applies, complete the Concept Outfall Evaluation section, attach the to the Concept Report, and add appropriate costs to the ROW and CST cost estimates. For more information regarding GDOT’s MS4 permit, please contact the Water Resources Group in the Office of Design Policy & Support at stormreports@dot.ga.gov

Is ecology water quality mitigation anticipated?   ☐ No  ☐ Yes

Coordinate with the Office of Environmental Services to determine if the project location and scope may require water quality design considerations for reasons other than MS4, such as protected species (i.e., aquatic species or predators of aquatic species) habitat mitigation, conservation area protection, etc. If ecology water quality mitigation is anticipated, add the anticipated costs to both the ROW and CST cost estimates. Consider how much of the project will need to be analyzed to adjust this percentage.

Will a Non-MS4 Detention Report be required during preliminary design?   ☐ No   ☐ Yes

Environmental Permits, Variances, Commitments, and Coordination anticipated: List all anticipated permits, variances, commitments, and coordination needed – Section 404, TVA, Water Quality, NPDES (e.g., NPDES Permit No. GAR100002 is needed due to land disturbance of an acre or more), etc.

Air Quality:

Is the project located in an Ozone Non-attainment or Maintenance area?   ☐ No   ☐ Yes

If “yes” to Ozone Non-attainment or Maintenance area, provide a comparison between the proposed project concept and the conforming plan’s model description. Include such features as project limits, number of through lanes, proposed open to traffic year, etc.  If project is exempt from conforming plan, explain why.

NEPA/GEPA Comments & Information:  Describe anticipated effects to ecology, history, archeology, air quality, noise effects, & the potential effect on the environmental document.

Public Involvement:  List level of Public Outreach expected including citizen committees, Public Information meetings, Public Hearings, Detour Meetings, etc.; also include any additional public outreach needed. For significant meetings previously completed, list dates, types of meetings, and attach meeting summaries or minutes.

Major stakeholders:  Identify major stakeholders in project (e.g., local government agencies, traveling public, business associations, neighborhood associations, etc.).

COORDINATION, ACTIVITIES, RESPONSIBILITIES, AND COSTS

Issues potentially affecting constructability/construction schedule:  Summarize any known issues which may affect the construction of the project (e.g., staging/detour issues, seasonal construction requirements, major traffic impacts, etc.)

Project meetings & other coordination to date: Include Concept Team Meeting and other significant project meetings that have been held in order by date held. Meeting minutes should be included in attachments.

| Project Activity | Party Responsible for Performing Task(s) |
| --- | --- |
| Concept Development | GDOT District number or Design Office, Consulting firm, local government, etc. |
| Design |  |
| Right-of-Way Acquisition |  |
| Utility Coordination (Preconstruction) |  |
| Utility Relocation (Construction) | typically performed by Utility Owners |
| Letting to Contract | GDOT or LOCL – if LOCL, include Letting agency* |
| Construction Supervision |  |
| Providing Material Pits |  |
| Providing Detours |  |
| Environmental Studies, Documents, & Permits |  |
| Environmental Mitigation |  |
| Construction Inspection & Materials Testing |  |

* For CID projects, the LLAF needs to be signed by the letting agency and attached to the concept report.

| Project Cost Estimate Summary and Funding Responsibilities: Add additional rows as necessary; Attach current cost estimates to report. Use latest Revisions to Programmed Costs template located on ROADS. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PE Activities |  | ROW | Reimbursable Utilities | CST* | Total Cost |
|  | PE Funding | Section 404 Mitigation |  |  |  |  |
| Date of Estimate: |  |  |  |  |  |  |
| Proposed Funding Source(s): |  |  |  |  |  |  |
| Programmed Cost: |  |  |  |  |  |  |
| Estimated Cost: |  |  |  |  |  |  |
| Total Cost Difference: |  |  |  |  |  |  |

*CST Cost includes Construction, Engineering and Inspection, Contingencies and Asphalt Fuel Price Adjustment.

Proposed Funding Source(s): show Federal, State, Local, or Undetermined (TBD) as applicable (i.e.  Federal, State, Federal/State, Federal/State/Local, TBD, etc.).  Please contact GDOT PM if you have any questions.

ROW, Utility, and CST estimates are to be included in attachments. Date of Estimate is the date the estimate was reviewed and/or approved. Estimates developed by design team are to be identified and noted below table (example: ** ROW Estimate developed by design team - submitted to GDOT for approval on xx/xx/xx). An estimated ROW cost should be included for each Alternative in the tables with any costs developed by the design team noted.

Total Cost Difference = Total Programmed cost vs Total Estimated Cost. If the total estimated cost is $2 Million or 20% greater than the total programmed cost, a brief explanation of the anticipated source of the additional required funding is needed (e.g., additional funding anticipated through SPLOST funds, additional federal or state funds will be pursued, etc.).

If Railroad PE and CONST costs are included in the PE Funding and Reimbursable Utilities, add note below table including estimated costs.

ALTERNATIVES DISCUSSION

Alternative selection: For Type I projects: Compare and contrast the various alternatives studied in summary and reason(s) why each alternative was or was not selected. Discussion should include no-build and preferred alternatives, and should compare various factors such as total cost, environmental and social impacts, time requirements, PE requirements, B/C ratio, etc. as appropriate to the decision process. If ROW, Utilities, and Mitigation costs have not been provided, the design team should estimate ROW costs and include a footnote under each applicable table (example: * Estimated ROW cost by design team). Add additional Alternative tables as necessary. For any alternatives that perform almost equally to the preferred alternative in terms of cost, impacts, and/or performance, prepare a high-level concept layout along with corresponding "decision-level" cost estimates sufficient for comparison to the preferred alternative.

For Type II projects: Only the Preferred Alternative table is required to be completed and the Rationale should describe how the project supports the specific goals of the project.

| Preferred Alternative: Provide a brief description of the alternative. |  |  |  |
| --- | --- | --- | --- |
| Estimated Property Impacts: |  | Estimated Total Cost: |  |
| Estimated ROW Cost: |  | Estimated CST Time: |  |
| Rationale: Reason(s) why this alternative was selected (operational improvement, cost, property and utility impacts, environmental impacts, crash reduction, etc.). Rationale should reflect how this alternative will address the issue(s) identified and goals defined in the Project Justification Statement (PJS), including data from supporting attachments that estimate improvements for opening and design years such as LOS, delay, and safety B/C ratio. For Type II projects complete only the Preferred Alternatives section. |  |  |  |

.

| No-Build Alternative: Provide a brief description of the alternative. |  |  |  |
| --- | --- | --- | --- |
| Estimated Property Impacts: |  | Estimated Total Cost: |  |
| Estimated ROW Cost: |  | Estimated CST Time: |  |
| Rationale: Reason(s) why this alternative was or was not selected |  |  |  |

| Alternative 1: Provide a brief description of the alternative. If similar to the preferred alternative, focus on describing the differences. For stand-along intersection projects, include all alternatives evaluated as part of ICE Stage 2. Copy and complete this table for any additional alternatives (Alternative 2, etc.). |  |  |  |
| --- | --- | --- | --- |
| Estimated Property Impacts: |  | Estimated Total Cost: |  |
| Estimated ROW Cost: | * required | Estimated CST Time: |  |
| Rationale: Reason(s) why this alternative was not selected (operational improvement, cost, property and utility impacts, environmental impacts, crash reduction, etc.). Rationale should provide a comparison to the preferred alternative and reflect why this alternative does not best address the issue(s) identified and goals defined in the PJS, including data from supporting attachments that estimate improvement for opening and design years such as LOS, delay, and safety B/C ratio for direct comparison to the preferred alternative. |  |  |  |

Add note if applicable - *Estimated ROW cost by design team.

Comments:  Add additional information as appropriate (e.g., other alternatives that were evaluated & eliminated and why, etc.). Briefly describe any major design decisions and coordination to be completed after concept approval.

LIST OF ATTACHMENTS/SUPPORTING DATA

List supporting data in the attached order, remove lines if not relevant. Cover pages & bookmarks for each attachment group are not required but may be useful for larger reports.

- Concept Layout – Preferred Alternative Layout should include north arrow, scale bar, legend, road and waterway names, estimated existing & required R/W, anticipated easements, property lines, ESA boundaries, and project begin and end labels as applicable. For longer projects, it may be helpful to include an overall project layout and provide break-out sheets with matchlines for clarity. For rural context, "less detail" should be the norm, which means that concept level cross-sections are seldom required. For suburban/urban context, "more detail" is often necessary, particularly where additional effort is needed to develop strategies for reducing footprint and/or impacts along a segment of roadway. For New Construction projects, concept level cross-sections cut/fill limits may be necessary to evaluate and minimize impacts to property and resources.

  - On-site Detour Layout If an on-site detour is required, include the detour/staging layout with pavement and lane shift tapers labeled.

- Typical sections Include conceptual roadway and bridge typical sections showing proposed lane & shoulder configurations, labels and pavement legend.

- Detailed Cost Estimates:

  - Construction Cost Estimate CST estimates should be generated using AASHTOWare Project Estimation and include Revisions to Programmed Costs and Cost Estimate Worksheets [see R.O.A.D.S.> Design Manuals> Design Related Resources> Engineering Services> Revisions to Programmed Costs template: [https://www.dot.ga.gov/GDOT/pages/designresources.aspx] The estimate should reflect the level of detail on the concept layout.  Specifically, it is OK to group minor cost items not shown on the layout and include the category as an “ad hoc” item (i.e., Lump Sum) in AASHTOWare Project Estimation.

  - Right-of-Way Attach GDOT ROW Cost Estimate Summary page; if ROW Cost Estimate was not developed by GDOT Office of ROW personnel, add footnote to ROW Cost Estimate Summary Page identifying author & organization. Do not include ROW cost worksheets.

  - Section 404 Mitigation Include cost estimates for 404 mitigation provided or approved by the Office of Environmental Services Special Projects Coordinator (currently Lisa Westberry)

  - Utilities Include Railroad costs if applicable; if RR costs were not developed by GDOT Utilities personnel, identify author & organization on cost estimate

- Concept Utility Report For GDOT Let projects the CUR is provided by District Utilities Office; for Local Let projects, the local government is responsible

- Crash summaries and diagrams For requirements, refer to the Concept Scoping Tool

- Traffic diagrams or projections If applicable

- Capacity analysis summary Tabular format recommended If applicable

- Summary of TE Study and/or Signal Warrant Analysis If applicable

- Stage 1 ICE Report(s) or Approved ICE Waiver If applicable – see ICE Policy

- Roundabout Data If applicable – see GDOT Design Policy Manual

- S I & A Report(s) Bridge/Structural Inventory Report(s) - If applicable

- MS4 Concept Report Summary If applicable – Only for projects within a designated GDOT MS4 (Municipal Separate Storm Sewer System) permit area

- Pavement studies E.g., Initial Pavement Type Selection Report

- Conforming plan’s network schematics showing thru lanes Required for capacity-adding projects in air quality non-attainment areas only; schematic is provided by Office of Planning Congestion Mitigation and Air Quality (CMAQ) unit

- Minutes of Concept Meetings

- Other Meeting Minutes - E.g., PIOH, PHOH, Detour, etc. that shows support or objection to the concept, Town Hall Meeting, etc.

- Signed Agreements Include Local Lighting Agreement if lighting is needed on GDOT route

- Other items referred to in the body of the report Delete this line if not used. Example of information to be included:  High-level concept layouts & other information on non-preferred alternatives with performance almost equal to the preferred alternative in terms of cost, impacts, and/or performance sufficient for comparison to the preferred alternative