PI #:

Project Description:

County:

SUE Consultant:

SUE Task Order # or Prime:

SUE LOS accepted date:

CAD Application:

The SUE Deliverables Checklist offers the current minimum requirements for GDOT SUE projects as an organized collection of information or a series of options.  This Checklist has been provided as a benefit to the SUE Provider and should be used in conjunction with professional judgment, education, and experience.  Not all items on the Checklist may be applicable in all circumstances as each project may have many unique aspects that should be considered.  This Checklist has been prepared in accordance with recognized engineering principles, standards, and practices.  It should not be used without the user’s competent knowledge of these principles, standards, and practices.  Satisfying this checklist does not guarantee acceptance of the SUE Deliverables as other comments may apply.

Definitions:

LOS = Limits of SUE Investigation

QL-D = Quality Level D SUE investigation

SUE = In Georgia: Overhead/Subsurface Utility Engineering

GDOT = Georgia Department of Transportation

EDG = Electronic Data Guidelines

SSUE = State Subsurface Utilities Engineer

SURE = Subsurface Utilities Review Engineer

Please indicate one of the following SUE Deliverable Submittals:

Initial	 Final

Instructions:

SUE Professional:

- Fill out the checklist in accordance with the SUE scope for the project

  - Ensure all items checked as ”included” have been appropriately shown on the SUE Deliverables

- Ensure all items scoped, but not found during the SUE investigation are appropriately checked as “excluded”

- Ensure all items not scoped and not found during the SUE investigation are appropriately checked as “excluded”

  - Provide details for all additions/exclusions in the space provided on the Signature Page

- Submit the SUE Deliverables Checklist to the SSUE/SURE

  - PDF only

  - Once with the Initial SUE Deliverables Submittal

    - Sign and date the Signature Page

  - Once with the Final Accepted SUE Deliverables Submittal

    - Certify (sign & seal) and date the Signature Page

- DO NOT SUBMIT INTERIM CHECKLISTS

The QL-D SUE Submittal will include the following information:

For each SUE Deliverable Submittal:

Included	Excluded

Provide all of the Microstation (version required per project’s scope) dgn files or image files used and created for the SUE investigation

For the final accepted SUE Deliverables Submittal:

Included	Excluded

Provide certified reproducible scanned images (black & white, PDFs) of the accepted SUE Deliverables

- Include a Georgia P.E. or R.L.S. stamp, signature, and date on the cover sheet only. At a minimum the SUE Consultant shall provide a Cover Sheet, General Notes Sheet and a Utility Legend Sheet. The SUE Consultant will be responsible for maintaining the original signed and sealed documents

Provide documentation of an accepted LOS (i.e. an email, LOS meeting minutes, etc.)

- The LOS must be accepted by the Designer and the SSUE prior to beginning the SUE investigation

# SUE Plan Set

# Cover Sheet (Sheet 24-0000)

Included	Excluded

Include a Key Map with:

- “Overhead/Subsurface Utility Engineering” in the title

- Available mapping and proposed design information (screened back)

- Sheet layout with drawing numbers (if available)

- Legible mainline and side street names

- The SUE LOS (clearly labeled/defined)

Provide a north arrow

Include a Title Block with:

- Task order number (if applicable)

- Project PI#

- County

- SUE Consultant Company name, address, Phone #

# General Notes Sheet (Sheet 24-000A)

# Included	Excluded

Provide any General Notes applicable to the SUE investigation

Include the following SUE note under title “SUE Investigation Description” (Fill in the appropriate items):

- “An Overhead/Subsurface Utility Engineering (SUE) Investigation was performed and completed by Consultant on MM/DD/YY for this project.  The existing overhead and underground facilities shown hereon within the LOS were included in this SUE Investigation.  The presence of these utility facilities has been investigated through records research and visual field inspections. Utilities that have been installed after the date above have not been included in this investigation.

- If mapping files are available include the following to the above note: All other existing topographic features depicted hereon have been referenced from a topographic/ mapping survey and control package provided by ___ and dated MM/DD/YY.”

Include a Utility Owners list in the plans showing: (see Appendix)

- Full name, acronym, and type of utility facility

- Include the GDOT and the Railroad as owners, when applicable

- A note specifying whether or not the Owner’s facilities were found within the LOS at the time of the SUE investigation

  - Include information indicating limits of ownership

- Indicate ROF (record on file) or RNA (record not available)

Include a Utility Owners list in deliverables checklist showing: (see Appendix)

- Full name, acronym, and type of utility facility

- Address

- Phone # of the “record” contact (the person actually contacted at each Utility Company who provided the record information)

- Include the GDOT and the Railroad as owners, when applicable

- A note specifying whether or not the Owner’s facilities were found within the LOS at the time of the SUE investigation

  - Include information indicating limits of ownership

- Indicate ROF (record on file) or RNA (record not available)

Include the following note above the Utility Owners list:

- “All the following Utility Owners were reported to have facilities within the vicinity of this project at the time of the SUE Investigation.  Utilities found within the LOS at the time of the SUE Investigation are noted for each Utility Owner listed and are shown on the plans hereon.”

Utility Legend Sheet (Sheet 24-000B)

Included	Excluded

Include the current SUE Utility Legend showing all of the following employed in the plans:

- Line Codes

- Symbols

- Abbreviations

- Quality Levels of SUE definitions

SUE Utility Plan Sheets

Providing cut sheets is optional for SUE QL-D deliverables and is a decision for the SUE Consultant.  If cut sheets are provided, the guidelines below shall be followed.  If cut sheets are not provided, check excluded for this section.

Included	Excluded

Provide cut sheets to cover the entire SUE investigation limits

- Use the most recent cut sheets for the Plans, if available

- Create cut sheets if the cut sheets for the Plans are not available

- Provide extra cut sheets if the SUE investigation limits extends beyond the cut sheet limits

Include all of the following:

- North Arrow

- Scale (as specified in Project’s SUE scope or as instructed by SSUE)

- Match lines

Provide a minimum of one label for the size and material type for all known underground utility

facilities within the LOS for each facility on each sheet, if available.

Provide a note on the cut sheet(s) stating that there were no existing utilities found during the SUE investigation within the LOS shown on that particular sheet, when applicable

SUE Files:  General Requirements

Included	Excluded

Ensure the electronic files containing the SUE information are in compliance with the current GDOT EDG Standards used for the project (refer to the sections regarding the Utility Plans/Existing Utilities).  This includes, but is not limited to:

- Levels

- Colors

- Line Weights

- Line Codes

- Reference files

  - Naming conventions

  - Logical names

  - Update Sequence (UTLE last)

Show the street names of mainline and side streets

Ensure that the existing utility information stands out and is legible

Show the accepted LOS

Clearly show all of the visible utility appurtenances and underground utilities within the LOS. If the mapping files are available used the surveyed utility appurtenances if provided.

Show the limits of ownership for all utilities.  This is required if there is more than one owner for a specific utility within the project’s limits.  This information may be derived from interviewing the respective utility owners, field verification, and/or utility records inspection

Graphically show the correct orientation and relative position of all utility symbols, as applicable

Provide sufficient labels for the size and material type for all known water (potable and non-potable), gravity sanitary sewer, sanitary force mains, gas, petroleum, and steam underground utility facilities within the LOS for each facility, if available.    Follow the guidelines below for electric, telecom, television, traffic control and ATMS.

- For Underground Electric: Clearly label all 3 phase facilities; clearly label all Transmission facilities as “X Transmission” (where “X”=Utility Owner acronym/name). (Do not provide gauges)

- For Overhead Electric: Clearly label Transmission facilities as “X Transmission” (where “X”=Utility Owner acronym/name)

- For Underground Telecom: Clearly label cable pair sizes/fiber optic strand sizes

- For Underground Television: Clearly label direct buried fiber optic strand sizes. (Do not provide coaxial sizes)

- For Underground Traffic Control: Clearly label direct buried fiber optic strand sizes

- For ATMS: Clearly label conduit information (i.e. 4-4” ducts or 4-4” conduits); clearly label all direct buried fiber optic strand sizes

If the size and material type is NOT available:

- For all utility mains: clearly label the line as “UNK SIZE/TYPE“

- For all utility services: clearly label the line as “SVC”, add the abbreviation to the Utility Legend and define as “Service, unknown size/type”

Provide size/material type for all fire lines, including the lines to the hydrants, if available from record information.  These lines should not be labeled as services.  It is acceptable to provide a general note regarding the fire hydrant lines, as applicable

Do not show Fire Hydrant valves. Per the current SUE Utility Legend, the Fire Hydrant Assembly includes the associated valve

Clearly indicate with sufficient labels or notation when the line sizes/material types change along a facility main, if available from record information

Show the extents, correct orientation and position of all large/atypical utility structures.  All dimensions will be shown on the plans (if available) and labeled accordingly.  Such structures may include, but are not limited to:

  - Telephone manholes

  - Water and electric vaults

Provide all duct run information available for all underground duct lines (power, communications, ATMS, etc.).  This may be derived from interviewing the respective utility owners or from utility records and includes information such as, but not limited to:

- Duct configuration

- Number of ducts/interducts (small conduit)

Show the extents of and label all Subscriber Loop Carrier (SLC) sites/Remote Terminals/Cell Tower sites within the LOS

SUE Plan Set:  Individual Utility Requirements

Sanitary Sewer

Included	Excluded

Show all sewers systems, including combination sewer

Provide SSMH ID’s where manhole top elevations and/or invert elevations are available from records.

Show pipe size, flow direction, and pipe material, if available from record information

Show the location and the corresponding connectivity of all lift stations that tie into the sewer network found within the LOS

Utility Poles / Overhead (OH) Utilities

Included	Excluded

Clearly show the aerial utilities found within the LOS

- Graphically ensure that the OH facilities do not interfere with the Underground facilities

- Include only Guy Wires for Guy Poles

- Pole guy anchors (optional)

Show the following required types of poles found within the LOS:

- Utility Poles (including non-private Light Poles and Transmission Poles/structures)

- Traffic Control Poles (including Pedestrian Signal Poles or Button Posts)

- OH Sign Strain Poles

- Highway Lighting Poles

- GDOT and Railroad Owned Poles

For any OH lines crossing the LOS:  Show aerial connectivity to the next nearest pole(s)/tower(s)/structure(s) outside of the LOS (outside the LOS to outside the LOS)

SUE Submittal Signatures:

Please indicate other project specific items as documented in the SUE contract, task order (if applicable):

Click here to enter text.

Please indicate basis for all excluded items below:

Click here to enter text.

All required elements have been included in the Final SUE deliverables.

SUE Professional (seal/signature as applicable)	Date

Please submit to:

Aisha Moultrie, P.E.

State Subsurface Utilities Engineer (SSUE)

GDOT Office of Utilities

One Georgia Center

600 W. Peachtree Street, 10th Floor

Atlanta, GA 30308

Fax.: 404-631-1934

Email: amoultrie@dot.ga.gov

APPENDIX

EXAMPLE UTILITY OWNER LIST ENTRIES TO BE INCLUDED IN PLANS

*Georgia Power Company (GPC) – Distribution

Note: All Distribution Power found within the LOS is Georgia Power unless otherwise noted on the plans. (ROF)

IGC Telecom (IGC) – Fiber Optic

Note: Utility Owner was contacted and verified that there are no facilities within the LOS of the project.

Newton County Water Authority (NCWA) – Water & Sewer

Note: Newton County has Water and Sewer within the LOS from the beginning of project to the Rockdale/Newton County Line. (RNA)

*Please ensure that a separate entry is provided for Georgia Power Transmission and Georgia Power Distribution.

EXAMPLE UTILITY OWNER LIST ENTRIES FOR CHECKLIST

*Georgia Power Company (GPC) – Distribution

Insert contact name

Insert contact address

Insert city, state, & zip

Tel. No.: Insert contact telephone number

Note: All Distribution Power found within the LOS is Georgia Power unless otherwise noted on the plans. (ROF)

IGC Telecom (IGC) – Fiber Optic

Insert contact name

Insert contact address

Insert city, state, & zip

Tel. No.: Insert contact telephone number

Note: Utility Owner was contacted and verified that there are no facilities within the LOS of the project. (RNA)