PI #:

Project Description:

County:

SUE Consultant:

SUE Task Order # or Prime:

The SUE Deliverables Checklist offers the current minimum requirements for GDOT SUE projects as an organized collection of information or a series of options.  This Checklist has been provided as a benefit to the SUE Provider and should be used in conjunction with professional judgment, education, and experience.  Not all items on the Checklist may be applicable in all circumstances as each project may have many unique aspects that should be considered.  This Checklist has been prepared in accordance with recognized engineering principles, standards, and practices.  It should not be used without the user’s competent knowledge of these principles, standards, and practices.  Satisfying this checklist does not guarantee acceptance of the SUE Deliverables as other comments may apply.

Definitions:

LOS = Limits of SUE Investigation

UIA = Utility Impact Analysis

SUE = In Georgia: Overhead/Subsurface Utility Engineering

GDOT = Georgia Department of Transportation

EDG = Electronic Data Guidelines

SSUE = State Subsurface Utilities Engineer

SURE = Subsurface Utilities Review Engineer

Please indicate one of the following SUE Deliverable Submittals:

Initial	 Final

Instructions:

SUE Professional:

- Fill out the checklist in accordance with the SUE scope for the project

- Submit the SUE Deliverables Checklist to the SSUE/SURE

  - Once with the Initial SUE Deliverables Submittal

    - Sign and date the Signature Page

  - Once with the Final Accepted SUE Deliverables Submittal

    - Certify (sign & seal) and date the Signature Page

- DO NOT SUBMIT INTERIM CHECKLISTS

Check Appropriate Project Type:

* Rural Project - Provide a completed “Utility Impact Analysis (UIA)” conflict matrix spreadsheet file (Excel or PDF format) All Fiber Optic, 2” or larger pipes, and 50 Pair or larger cables*

* Urban Project - Provide a completed “Utility Impact Analysis (UIA)” conflict matrix spreadsheet file (Excel or PDF format) All Fiber Optic, 4” or larger pipes, and 200 Pair or larger cables*

** PID Project - Provide a completed “Utility Impact Analysis (UIA)” conflict matrix spreadsheet file (Excel or PDF format) All conflicts must be included in the analysis.

* Note: Only include underground conflicts, no above ground appurtenances.

** Note: Include Poles, manholes, vaults, and above ground appurtenances. Do not include pedestrian poles, service meters, valves, or pull-boxes.

The UIA SUE Submittal will include the following information:

For each SUE Deliverable Submittal:

* Note: Ensure to follow the instructions provided in the UIA (conflict matrix) spreadsheet.

Included	Excluded

Provide the following files:

  - Create a new reference file called 1234567_UIA.dgn (where 1234567 = the PI#).  This file will only contain the UIA Conflict Locations.  Ensure each conflict between an existing utility facility and a proposed design feature is marked in this file.

  - Provide a completed “Utility Impact Analysis (UIA)” conflict matrix spreadsheet file (Excel format)

- A downloadable conflict matrix called “SampleBlankConflictMatrix” can be found on the current GDOT Utilities site:

- Fill out the following project information (shown at the top of the spreadsheet)

- Project Description

- County

- Project PI#

- Consultant Company name

- Date

Create a PDF file called 1234567_UIA. (where 1234567 = the PI#).  This file will show the 				conflict circles in the utility plans. Label the utility and the conflict for each plan sheet. If there are 			multiple utilities, list each utility.

- This will typically apply when there are minimal conflict and no utility congestion.

- Conflict points and information must be legible in pdf

Provide all of the Microstation (version required per project’s scope) dgn files used and created in 			determining the utility conflicts. The minimum required proposed design files are listed below.  			Ensure all of these files are available prior to beginning the UIA.  Contact the SSUE or 			designer if these files are not available.

- Cross-sections

- Plans (roadway, bridge, wall, traffic control, lighting plans, staging) if available

- Roadway Profiles

- Drainage (at minimum in plan view)

For the final accepted SUE Deliverables Submittal:

Provide 11 x17 color PDFs and excel spreadsheet of the accepted “Utility Impact Analysis (UIA)” 			conflict matrix spreadsheet

- Include a Georgia P.E. or R.L.S. stamp, signature, and date on 1st sheet only and checklist

and/or

Provide 11 x 17 black and white PDFs of the plans with the cover sheet

- Include a Georgia P.E. or R.L.S. stamp, signature, and date on cover sheet only and checklist

SUE Submittal Signatures:

Please indicate other project specific items as documented in the SUE contract, task order (if applicable):

Click here to enter text.

Please indicate basis for all excluded items below:

Click here to enter text.

All required elements have been included in the Final SUE deliverables.

SUE Professional (seal/signature as applicable)	Date

Please submit to:

Aisha Moultrie, P.E.

State Subsurface Utilities Engineer (SSUE)

GDOT Office of Utilities

One Georgia Center

600 W. Peachtree Street, 10th Floor

Atlanta, GA 30308

Fax.: 404-631-1934

Email: amoultrie@dot.ga.gov